## Complete DSP Lab Guide — main_radar.m

#### Written as if your lab instructor is sitting next to you

## BEFORE WE START — The Big Picture

Think of your entire main_radar.m as a factory production line. Raw material goes in one end and a finished product comes out the other.

Raw material:    You decide what pulse to transmit

↓

Step 1:          Generate the transmitted waveform

↓

Step 2:          Simulate what the receiver gets back

↓

Step 3:          Clean the signal (remove clutter)

↓

Step 4:          Find the targets (matched filter)

↓

Step 5:          Prepare for frequency analysis (windowing)

↓

Step 6:          Make the detection decision (CFAR)

↓

Finished product: "Target at 500m, 800m. Pd = 0.85"

Every section of the code is one step on that production line. If one step breaks or is set up wrong, everything downstream suffers. Keep this factory analogy in your head throughout.

## SECTION 1 — Fixed System Parameters

matlab

fs     = 10e6;    % 10 MHz

T      = 10e-6;   % 10 microseconds

B      = 1e6;     % 1 MHz

c      = 3e8;     % speed of light

SNR_dB = 10;      % 10 dB

N_fft  = 1024;

#### What is fs — Sampling Frequency?

Your radar receives an analogue signal — a continuous wave that changes every nanosecond. A computer cannot store infinite points so you take snapshots at regular intervals. fs = 10 MHz means you take 10 million snapshots every second.

Why does this matter? The Nyquist theorem says you must sample at least twice as fast as the highest frequency in your signal. Our signal bandwidth is 1 MHz so the absolute minimum is 2 MHz. We use 10 MHz which gives us comfortable headroom and finer range resolution.

Effect on code: Every time you see /fs in the code it is converting from sample numbers to time. Every time you see * c / (2 * fs) it is converting samples to metres. Change fs and every distance measurement changes.

matlab

cell_size = c / (2 * fs) = 3e8 / (2 × 10e6) = 15 metres per sample

So each sample in your matched filter output represents exactly 15 metres of range.

#### What is T — Pulse Duration?

This is how long you hold the transmit button down. 10 microseconds means you transmit for 10 millionths of a second.

The fundamental conflict:

- Longer pulse = more energy transmitted = stronger echo = easier to detect

- Longer pulse = you cannot detect targets closer than c×T/2 = 1500m (blind zone)

- Shorter pulse = better range resolution but weaker echo

This is why LFM was invented — transmit a long pulse to get energy, then compress it to get resolution. Best of both worlds.

Effect on code: pulse_length = round(T × fs) = 10×10⁻⁶ × 10×10⁶ = 100 samples Every waveform generated is exactly 100 samples long. The receive buffer is sized relative to T.

#### What is B — Bandwidth?

Bandwidth is how wide a range of frequencies your pulse sweeps across. For LFM, if you start at 1 GHz and end at 1.001 GHz, your bandwidth is 1 MHz.

Why bandwidth matters: Range resolution = c / (2×B) = 3×10⁸ / (2×10⁶) = 150 metres

This means two targets must be at least 150m apart for you to see them as two separate peaks. Put them closer and they merge into one blob.

Effect on code: B appears directly in generate_LFM and generate_Hybrid. Change B and the chirp rate k = B/T changes. Smaller B = slower sweep = wider peaks = worse resolution. Larger B = faster sweep = narrower peaks = better resolution.

#### What is TBP — Time Bandwidth Product?

TBP = T × B = 10×10⁻⁶ × 1×10⁶ = 10

This single number tells you how much SNR gain you get from matched filtering. Our TBP of 10 means after matched filtering the SNR improves by 10 dB. This is why your output shows "SNR before: 10 dB, SNR after: ~16-18 dB" — the matched filter is doing its job.

Think of it like this — you shout a 10 second message into a noisy room. Someone records it all 10 seconds. They know exactly what you said so they can match the recording against a template and pull your voice out of the noise much more cleanly than if you had only spoken for 1 second. TBP is essentially how many seconds you shouted.

#### What is SNR_dB?

Signal to Noise Ratio in decibels. This is a ratio between signal power and noise power.

SNR = 10 dB means signal power is 10× noise power

SNR = 0 dB  means signal and noise are equal power

SNR = -10 dB means noise is 10× stronger than signal

In the code, SNR controls how much noise is added:

matlab

noise_power = signal_power / 10^(SNR_dB/10)

Lower SNR = more noise added = harder to detect = lower Pd. This is your stress test dial.

## SECTION 2 — User Inputs

#### How validatestring Works

matlab

waveform_type = validatestring(upper(strtrim(input(...))), {'LFM','PHASECODE','HYBRID'});

Three things happen here in order:

- input(...) — waits for you to type something

- strtrim(...) — removes any accidental spaces you typed

- upper(...) — converts to uppercase so lfm = LFM

- validatestring(...) — checks your input against the allowed list

If you type lf it matches LFM because validatestring does partial matching. If you type xyz it throws a clean error immediately rather than crashing 200 lines later with a confusing message.

This matters in your viva — if you demo the code and accidentally type wrong the code should not crash spectacularly.

#### The Swerling Model Input

matlab

swerling_model = input('Enter Swerling model (0/1/2/3/4): ');

if ~isscalar(swerling_model) || ~ismember(swerling_model, 0:4)

error('Swerling model must be 0, 1, 2, 3, or 4');

end

This uses numeric input not string so validatestring is not used here. Instead:

- isscalar checks you entered one number not a vector

- ismember(x, 0:4) checks it is between 0 and 4

- If you type 5 or -1 you get a clean error immediately

## SECTION 3 — Waveform Generation

matlab

switch waveform_type

case 'LFM',       transmitted = generate_LFM(T, B, fs);

case 'PhaseCode', transmitted = generate_PhaseCode(fs, T);

case 'Hybrid',    transmitted = generate_Hybrid(T, B, fs);

end

pulse_length = length(transmitted);

#### What is Actually Happening Here?

You are creating the mathematical representation of the electromagnetic pulse your radar will transmit. The output transmitted is an array of 100 complex numbers — each number represents the amplitude and phase of the signal at that instant in time.

Why complex numbers? Because real radar signals have two components — in-phase (I) and quadrature (Q). Together they form a complex number. This is standard in all DSP.

#### LFM — generate_LFM(T, B, fs)

Inside the function:

matlab

t = 0 : 1/fs : T - 1/fs;    % time vector: 100 points

k = B / T;                    % chirp rate = 1e11 Hz/s

signal = exp(1j * pi * k .* t.^2);

The formula exp(jπkt²) is the mathematical definition of a chirp. At t=0 the frequency is zero. At t=T the frequency has swept up by B Hz. The phase increases quadratically with time which is what causes the frequency to increase linearly.

What it looks like: If you plot the real part you see a wave that starts slow and cycles faster and faster — like a siren speeding up.

Key parameter: k = B/T is the chirp rate. It controls how fast the frequency changes. Our k = 10⁶/10⁻⁵ = 10¹¹ Hz per second. Fast sweep = wide bandwidth = good resolution.

#### PhaseCode — generate_PhaseCode(fs, T)

Inside the function:

matlab

barker13 = [1 1 1 1 1 -1 -1 1 1 -1 1 -1 1];

The signal is divided into 13 equal chips. Each chip is either +1 (normal phase) or -1 (flipped phase). The pattern is the Barker-13 sequence which was mathematically proven to give the best possible sidelobe suppression for a 13-chip binary code.

What it looks like: If you plot the real part you see a flat square wave that occasionally flips sign — like morse code.

Why 13 chips specifically? Barker codes only exist for lengths 1, 2, 3, 4, 5, 7, 11, and 13. Length 13 gives the best compression ratio available in the Barker family. No Barker code longer than 13 has ever been found — it is an open mathematical problem.

#### Hybrid — generate_Hybrid(T, B, fs)

Instead of each chip being a flat constant, each chip is a mini LFM chirp. The Barker code still modulates the chips but now each chip has frequency structure inside it.

matlab

chip = barker13(i) * exp(1j * pi * k .* t_chip.^2);

The key fix in our code is that we calculate samples_per_chip = floor(total_samples / num_chips) first and pre-allocate a signal of exact length. The old version tried to trim afterwards and crashed because rounding errors made it 1 sample too short.

## SECTION 4 — Simulating the Received Signal

This is the most complex section. You are building the fake world that your radar is operating in.

#### Why Simulate?

You have no real transmitter, no real aircraft, no real receiver. So you construct mathematically what the receiver would see if those things existed. This is called a controlled simulation and it is standard practice in all radar research because:

- You know the ground truth so you can verify detection accuracy

- You can test extreme conditions safely

- You can repeat experiments exactly

#### Step 1 — Receive Buffer

matlab

max_delay_samples = round(2 * max(target_ranges) * fs / c);

receive_length    = pulse_length + max_delay_samples + round(fs * 50e-6);

received          = zeros(1, receive_length);

You create an array of zeros — an empty container. You need it long enough to hold the echo from the furthest target plus some extra buffer.

The 2 * is because the signal travels to the target AND back — double the distance.

round(fs * 50e-6) adds 500 extra samples as buffer so echoes arriving slightly later than expected do not get cut off.

#### Step 2 — Swerling Amplitude

matlab

if swerling_model == 0

actual_amp = base_amp;

elseif swerling_model == 1 || swerling_model == 2

actual_amp = base_amp * abs(randn + 1j*randn) / sqrt(2);

elseif swerling_model == 3 || swerling_model == 4

actual_amp = base_amp * abs(randn + 1j*randn + randn + 1j*randn) / 2;

end

Swerling 0: actual_amp = base_amp. No randomness. Every run gives the same result.

Swerling 1/2 — Rayleigh distribution: abs(randn + j·randn) generates a complex Gaussian random variable. Its magnitude follows the Rayleigh distribution. Dividing by √2 keeps the average power equal to the base amplitude.

Rayleigh models a target with many small independent scatterers — like an aircraft with rivets, panels, engines all reflecting together. Sometimes they add up, sometimes they cancel. The amplitude fluctuates randomly around the mean.

Swerling 3/4 — Chi-squared with 4 degrees of freedom: abs(randn+j·randn + randn+j·randn) / 2 is the sum of two complex Gaussians. This models a target with one dominant scatterer plus many small ones — like a missile where the nose cone dominates the reflection. Less extreme fluctuations than Rayleigh.

Slow vs fast (1/3 vs 2/4): In our simulation both appear the same because we only generate one value per target per run. In a real multi-pulse radar, slow fluctuation means the amplitude is constant within one scan (say 10 pulses) then jumps. Fast fluctuation means it changes every single pulse. Our code demonstrates the statistical distribution — you would need to add a pulse loop to show the time behavior properly.

#### Step 3 — Doppler Shift

matlab

doppler_shift = 2 * target_velocities(i) * fc / c;

t_pulse_vec   = (0 : pulse_length-1) / fs;

doppler_phase = exp(1j * 2*pi * doppler_shift * t_pulse_vec);

echo          = actual_amp * transmitted .* doppler_phase;

What is happening physically? When a source moves towards you, each successive wave crest travels a slightly shorter distance and arrives slightly sooner than the previous one. This compresses the wave — higher frequency. Moving away stretches the wave — lower frequency. This is the Doppler effect.

The formula: doppler_shift = 2 × velocity × fc / c

The factor 2 is because the wave travels to the target AND back — the relative motion affects both legs of the journey. fc = 1 GHz is the carrier frequency. c = 3×10⁸.

Example: Aircraft at 200 m/s towards radar: doppler = 2 × 200 × 10⁹ / 3×10⁸ = 1333 Hz

The echo comes back 1333 Hz higher than what was transmitted. The FFT output (Plot 7) would show this shift.

In the code: exp(j2π·f_d·t) is a complex exponential that rotates in the complex plane at frequency f_d. Multiplying the echo by this rotates every sample — this is frequency shifting.

#### Step 4 — Range-Doppler Coupling (LFM Only)

matlab

if strcmp(waveform_type, 'LFM') && target_velocities(i) ~= 0

range_error_m = target_velocities(i) * T * c / (2 * B);

range_error_samples = round(range_error_m * 2 * fs / c);

end

delay_samples = round(2 * target_ranges(i) * fs / c) + range_error_samples;

Why does LFM have this problem? LFM compresses the echo by correlating it with the reference. The correlation finds the delay where they best match. If the echo is Doppler-shifted, its frequency is slightly different from the reference. But in LFM, a frequency shift looks identical to a time delay — because frequency and time are coupled in the chirp. Result: the target appears at the wrong range.

The formula: range_error = velocity × T × c / (2B)

With velocity=200 m/s, T=10µs, B=1 MHz: error = 200 × 10⁻⁵ × 3×10⁸ / (2×10⁶) = 0.3 metres. Very small in our parameters but with higher velocity or narrower bandwidth it becomes significant.

Why PhaseCode does not have this: Phase coding does not depend on matching a frequency sweep. The compression is based on matching the phase sequence. Doppler shifts the frequency slightly but does not change the phase code pattern significantly at normal target velocities.

#### Step 5 — AWGN Noise

matlab

signal_power = mean(abs(received).^2);

noise_power  = signal_power / 10^(SNR_dB/10);

noise        = sqrt(noise_power/2) * (randn(1,receive_length) + 1j*randn(1,receive_length));

received     = received + noise;

What is AWGN? Additive White Gaussian Noise. Every electronic component produces random thermal noise. It is:

- Additive — simply added to whatever signal is there

- White — equal power at all frequencies (like white light containing all colors)

- Gaussian — the amplitude distribution follows a bell curve

The code: 10^(SNR_dB/10) converts dB to linear ratio. noise_power = signal_power / ratio gives noise the right power. Complex noise uses two randn calls for I and Q components. Dividing by sqrt(2) normalises the complex power correctly.

#### Step 6 — Time-Varying Clutter

matlab

amp_drift1  = 0.05 + 0.03 * sin(2*pi*0.5 * t_recv);

freq_drift1 = 0.04e6 + 0.01e6 * sin(2*pi*0.2 * t_recv);

clutter1    = amp_drift1 .* exp(1j * 2*pi * cumsum(freq_drift1) / fs);

What is clutter physically? Ground returns, weather, buildings — they all reflect radar energy. Unlike noise which is random, clutter has structure — it comes from specific directions and has consistent frequency characteristics.

Why time-varying? Real clutter is not static. Rain intensifies and weakens. Sea waves change speed. Wind moves vegetation. So the clutter amplitude and frequency slowly drift over time.

How the code does it:

- amp_drift oscillates slowly between 0.02 and 0.08 — amplitude breathes over time

- freq_drift oscillates around a centre frequency — clutter shifts in frequency slowly

- cumsum(freq_drift) accumulates phase — this is how frequency-varying signals are built correctly. The phase at each sample is the running sum of all previous frequencies

- Three components at different centre frequencies (40 kHz, 70 kHz, 20 kHz) represent different clutter sources

Why such small amplitudes (0.05)? We learned the hard way — clutter amplitudes of 0.3 were overwhelming weak targets completely. Realistic clutter should be present but not dominant.

## SECTION 4.5 — Clutter Mitigation

matlab

received_raw = received;   % save original for plots

if strcmp(use_MTI, 'yes'),      received = apply_MTI(received);         end

if strcmp(use_adaptive, 'yes'), received = apply_adaptive(received);    end

if strcmp(use_doppler, 'yes'),  received = apply_doppler(received, fs); end

We save received_raw before any filtering so Plot 2 can show the dirty original while Plot 3 shows the cleaned version. The comparison between these two plots is one of the most important visual demonstrations in your CEP.

#### MTI — Moving Target Indicator

The core idea: Clutter does not move. Targets do. Subtract consecutive pulses — stationary things cancel, moving things survive.

The 3-pulse canceller:

output(n) = received(n) - 2×received(n-1) + received(n-2)

This is equivalent to applying a filter with coefficients [1, -2, 1]. In Z-transform: H(z) = 1 - 2z⁻¹ + z⁻². This has a zero at z=1 which is DC (zero frequency) — meaning stationary clutter at zero Doppler gets multiplied by zero — it disappears.

Why 3-pulse and not 2-pulse? The 2-pulse canceller (output = x(n) - x(n-1)) also works but the 3-pulse canceller has a double zero at DC giving much stronger clutter suppression — the null is deeper.

Blind speeds: If a target is moving at exactly the right velocity, its Doppler frequency hits another zero of the filter and it also gets cancelled. These are called blind speeds. They are a fundamental limitation of MTI.

#### Adaptive LMS Filter

The core idea: Learn what the clutter looks like, subtract your learned estimate, keep the residual (which should be mostly target).

matlab

for n = filter_len : N

x                = received(n:-1:n-filter_len+1);  % last 8 samples

clutter_estimate = weights * x.';                   % predict clutter

error_signal     = received(n) - clutter_estimate;  % subtract

weights          = weights + mu * error_signal * conj(x);  % update

output(n)        = error_signal;

end

Step by step:

- Take the last 8 samples as context (the filter memory)

- Multiply each by a learned weight and sum — this is your clutter prediction

- Subtract prediction from actual — error signal is what remains

- Update weights using the error — move slightly in direction that reduces error

- Repeat for every sample — filter converges over time

mu = 0.001: This is the learning rate. Large mu = learns fast but unstable. Small mu = learns slowly but safely. We use 0.001 to avoid accidentally learning to cancel the target echo — we want it to only learn the clutter pattern which evolves slowly.

Why LMS is better than MTI for some scenarios: MTI only kills zero-Doppler clutter. Adaptive filter learns whatever clutter is present — even clutter with non-zero Doppler like weather moving with wind, or sea clutter with wave motion. This is what the CEP means by "non-stationary clutter."

#### Doppler Filter

matlab

notch_hz   = 2000;

notch_bins = round(notch_hz * N / fs);

fft_received(1 : notch_bins)       = 0;

fft_received(N-notch_bins+1 : end) = 0;

output = ifft(fft_received, N);

Steps:

- FFT the received signal — go to frequency domain

- Zero out ±1000 Hz around zero (the DC region where stationary clutter lives)

- IFFT back to time domain — clutter gone

The critical fix we made: The old code used a 2% fraction of fs which gave a 200,000 Hz notch — enormous. It wiped out targets with Doppler shifts of only hundreds of Hz. The new code uses a fixed 2000 Hz notch. Stationary clutter has zero Doppler. A target moving at even 1 m/s produces 2×1×10⁹/3×10⁸ = 6.7 Hz Doppler — safely outside the 2000 Hz notch.

Order of filters: MTI first (kills bulk clutter fast), then adaptive (refines residual), then Doppler (final frequency domain cleanup). Each layer removes what the previous one missed.

## SECTION 5 — Matched / Mismatched Filtering

#### The Matched Filter Concept

You know exactly what you transmitted. When the echo comes back, you ask: "does any part of this noisy received signal look like what I transmitted?" You slide a copy of the transmitted signal across the received signal and at every position compute how well they match. Maximum match = target location.

This is mathematically cross-correlation.

#### The Three Filter Types

Matched (reference = transmitted exactly):

matlab

reference = transmitted;

mf_output = abs(xcorr(received, reference));

Maximum possible SNR gain. The matched filter theorem proves no linear filter can do better than correlating with the exact transmitted signal.

Mismatched BW (reference has 90% of bandwidth):

matlab

B_mismatch = B * 0.90;

reference  = generate_LFM(T, B_mismatch, fs);

The reference sweeps 10% slower than the actual echo. When you correlate them they do not align perfectly — the peak is slightly lower (SNR loss ~1-2 dB) and slightly wider (worse resolution). This simulates what happens when your radar hardware drifts slightly from its calibrated frequency.

Mismatched Window (Hamming-windowed reference):

matlab

reference = transmitted .* hamming(ref_len)';

You deliberately apply a Hamming window to the reference before correlating. Because the reference now has smooth edges, the correlation sidelobes are much lower. But the centre of the reference is also reduced in amplitude so the peak height drops — detection loss of ~1.3 dB. You are trading SNR for cleaner sidelobes. This is actually done intentionally in real radar systems for high-resolution imaging.

#### xcorr vs conv

matlab

% xcorr method

mf_raw    = abs(xcorr(received, reference));

mf_output = mf_raw(ceil(end/2) : end);   % keep positive lags

% conv method

mf_raw    = abs(conv(received, conj(fliplr(reference))));

mf_output = mf_raw(1 : length(received));

Why are they equivalent? Cross-correlation of x with y is defined as: R(τ) = ∫x(t)·y*(t+τ)dt Convolution of x with y_flipped is: ∫x(t)·y*(τ-t)dt

When you flip y and conjugate it, convolution and cross-correlation produce the same result. Flipping in time domain = reversal of the signal. Conjugating flips the imaginary part.

The length difference:

- xcorr output length = 2N-1 (runs from negative to positive lags) — we take the second half

- conv output length = N1+N2-1 — we take the first N samples

Both give the same peak at the same range location.

#### SNR Measurement

matlab

peak_val      = max(mf_output);

noise_samples = min(round(length(mf_output)*0.2), length(mf_output));

noise_floor   = mean(mf_output(end-noise_samples+1 : end));

snr_after     = 20 * log10(peak_val / max(noise_floor, eps));

Why last 20% for noise floor? The xcorr output stretches far beyond the actual target ranges. The last 20% of the output is at ranges well beyond any target — it contains only noise and residual clutter. This gives an accurate noise floor estimate.

The old code used mf_output(1:100) — the first 100 samples — which are near zero range and contain correlation edge effects that are much higher than the true noise floor, inflating the estimate and making SNR appear lower than it really is.

max(noise_floor, eps) prevents divide by zero if noise_floor is somehow zero.

20 × log10 — why 20 not 10? When converting amplitude ratios to dB you use 20. When converting power ratios to dB you use 10. Since mf_output is amplitude (not power), we use 20.

## SECTION 6 — Windowing and FFT

#### Why Window Before FFT?

The matched filter gives you a signal in time domain — a series of peaks at different ranges. FFT converts this to frequency domain to reveal Doppler information (target velocities).

The problem: FFT mathematically assumes your signal repeats forever. But your signal has a definite start and end. This abrupt cut creates sharp edges. Sharp edges contain energy at ALL frequencies — this is spectral leakage. Without windowing, a strong target's energy leaks into neighbouring frequency bins and masks weak targets.

The solution: Multiply by a window function that smoothly fades the signal to zero at both edges before FFT sees it. No sharp edges = no spectral leakage.

#### The Four Windows in the Code

matlab

switch window_type

case 'rectangular', w = ones(mf_len,1);   sidelobe_ref = -13;

case 'hamming',     w = hamming(mf_len);   sidelobe_ref = -43;

case 'hanning',     w = hann(mf_len);      sidelobe_ref = -31;

case 'blackman',    w = blackman(mf_len);  sidelobe_ref = -58;

end

mf_windowed = mf_output .* w';

Rectangular (-13 dB): Multiply by 1 everywhere — no change. Full resolution but terrible sidelobes. Only useful as baseline comparison.

Hanning (-31 dB): w(n) = 0.5×(1-cos(2πn/N)). Goes to zero at both edges. Good general purpose. Moderate resolution loss.

Hamming (-43 dB): w(n) = 0.54-0.46×cos(2πn/N). Stops at 0.08 at edges (not zero). The 0.08 was mathematically optimised to minimise the peak sidelobe. Better than Hanning at sidelobe suppression with similar resolution. This is why it is our default.

Blackman (-58 dB): Uses two cosine terms in its formula. Widest taper. Best sidelobes but worst resolution. Use when you have a strong target masking a weak nearby target.

The key insight about resolution: Resolution is not about where the edges stop. It is about how wide the flat middle section is. Hamming has a flatter, wider middle section than Hanning so it preserves more signal energy in the centre — better resolution despite stopping at 0.08 instead of 0.

#### The FFT

matlab

fft_mag   = abs(fft(mf_windowed, N_fft));

fft_mag   = fft_mag(1 : N_fft/2);              % positive frequencies only

fft_dB    = 20 * log10(fft_mag/max(fft_mag) + eps);

freq_axis = (0 : N_fft/2-1) * (fs/N_fft) / 1e6;

N_fft = 1024: Zero-pads the signal to 1024 points. Zero-padding does not add information but gives finer interpolation of the frequency axis — smoother looking spectrum. The signal is only 100-500 samples; padding to 1024 fills the rest with zeros.

Why keep only first half: FFT of a real-valued signal is symmetric — the second half is a mirror of the first. Taking only the first N/2 points gives you the positive frequencies. The negative frequency half is redundant. Since our signal is complex-valued (I+jQ) the full spectrum is used, but we still only display positive frequencies for clarity.

Frequency axis calculation: f = sample_index × (fs/N_fft). The frequency resolution is fs/N_fft = 10MHz/1024 ≈ 9.8 kHz per bin. Dividing by 1e6 converts to MHz for the plot axis.

+ eps before log10: eps is the smallest positive floating point number in MATLAB (~2.2×10⁻¹⁶). Adding it prevents log10(0) = -infinity which would cause plotting errors.

## SECTION 7 — Detection

#### The Four Parameters

matlab

num_train    = 8;    % training cells each side

num_guard    = 2;    % guard cells each side

alpha        = 2.0;  % threshold scaling factor

k_os         = 12;   % OS-CFAR rank

fixed_factor = 3.0;  % fixed threshold multiplier

num_train = 8: For each candidate cell (CUT), you look at 8 cells to the left and 8 cells to the right to estimate local noise. Total of 16 training cells. More cells = more accurate estimate but slower and includes noise from farther away.

num_guard = 2: Two cells on each side of the CUT are excluded from the training region. They act as a buffer. Why? The matched filter response has a main peak but energy spills into adjacent cells. Without guard cells this spillover would inflate the noise estimate and raise the threshold high enough to miss the target itself.

alpha = 2.0: The threshold = alpha × noise_estimate. Alpha = 2.0 means the threshold is set at twice the estimated noise level. Lower alpha = lower threshold = more detections but more false alarms. Higher alpha = fewer false alarms but more missed targets. We reduced from 3.0 to 2.0 because our clutter has been well controlled and we want reasonable detection sensitivity.

k_os = 12: For OS-CFAR with 16 total training cells, k=12 means we take the 12th value from the sorted list. This is the 75th percentile — high enough to represent the noise floor but below the top 25% which might contain interfering targets.

#### The Detection Switch

matlab

switch detect_method

case 'fixed'

[detected_indices, threshold_line] = detect_fixed(mf_output, fixed_factor);

case 'ca'

[detected_indices, threshold_line] = detect_CA_CFAR(mf_output, num_train, num_guard, alpha);

case 'os'

[detected_indices, threshold_line] = detect_OS_CFAR(mf_output, num_train, num_guard, alpha, k_os);

case 'go'

[detected_indices, threshold_line] = detect_GO_CFAR(mf_output, num_train, num_guard, alpha);

end

Note that detect_method was stored as lowercase (ca not CA) so the switch cases use lowercase. The upper(detect_method) in the print statements is only for display.

#### Clustering

matlab

for i = 2 : length(detected_indices)

if detected_indices(i) - detected_indices(i-1) <= 3

cluster = [cluster, detected_indices(i)];

else

clusters{end+1} = cluster;

cluster = detected_indices(i);

end

end

Why cluster? A single real target produces a peak that is several cells wide — the matched filter response has non-zero width. CFAR might flag cells 33, 34, and 35 as above threshold even though they all correspond to the same target at cell 33. Without clustering you would report three detections when only one target exists.

How it works: If two detected indices are within 3 samples of each other (3 × 15m = 45m) they are grouped into the same cluster. The peak cell within each cluster is reported as the target location.

#### Pd and Pfa Calculation

matlab

target_matched    = false(1, num_targets);

detection_matched = false(1, num_det);

for i = 1 : num_targets

idx = find(abs(detected_ranges - target_ranges(i)) <= tolerance_m ...

& ~detection_matched, 1, 'first');

if ~isempty(idx)

target_matched(i)      = true;

detection_matched(idx) = true;

end

end

true_detected    = sum(target_matched);

false_detections = num_det - sum(detection_matched);

Pd               = true_detected / max(num_targets, 1);

Pfa              = false_detections / max(length(mf_output) - num_targets, 1);

Why boolean arrays? The old code had a bug — if two detections landed near the same target, both would be counted as correct. The boolean arrays prevent this. Once a target has been matched (target_matched(i) = true) it cannot be matched again. Once a detection has been used (detection_matched(idx) = true) it cannot be reused.

tolerance_m = 50: A detection within 50m of the true range is counted as correct. 50m is approximately 3 range cells (3×15m=45m) which accounts for small timing errors in the peak location.

Pfa denominator: length(mf_output) - num_targets. Total empty cells = total cells minus the cells that actually contain targets. False alarm rate = false detections per empty cell.

## SECTION 8 — Plots

#### Figure 1 (8 subplots)

Plot 1 — Transmitted signal (time domain): LFM shows a chirp — wave oscillating faster and faster. PhaseCode shows a square-wave pattern with occasional phase flips. Hybrid shows something in between. If this looks wrong, check waveform generation.

Plot 2 — Received before mitigation: Should look very messy — noise and clutter dominate. Targets are invisible here. This is the realistic input to your processing chain.

Plot 3 — Received after mitigation: Should look cleaner. If it looks identical to Plot 2 your filters had no effect — check that you enabled them. If it looks completely flat your filters were too aggressive and destroyed everything including targets.

Plot 4 — MF output full range: This is the money plot. You should see one or more clear peaks rising above the noise floor. Green dashed lines mark the true target positions. Peaks should align with green lines. If peaks are shifted slightly, that is range-Doppler coupling.

Plot 5 — MF output zoomed: Same as Plot 4 but zoomed into the region around your targets. Makes it easier to see if two close targets are resolved as two peaks or merged into one.

Plot 6 — SNR bar chart: Right bar should always be taller than left bar for a functioning matched filter. The height difference is your SNR gain. Theoretically should be close to 10 dB (our TBP). If right bar is lower, something went wrong — clutter filters or mismatch damaged the signal too much.

Plot 7 — FFT spectrum: The red dashed line shows the expected sidelobe level for your chosen window. The actual spectrum should have sidelobes at or below that line. If sidelobes are above the line something is wrong with the windowing. The X axis is frequency in MHz — Doppler-shifted targets would appear as peaks offset from zero.

Plot 8 — Normalised MF output in dB: The main peak is normalised to 0 dB. Everything else shows relative sidelobe levels. You want the noise floor well below -13 dB. The sidelobe bumps near the main peak should match what your chosen window promises.

#### Figure 2 — Detection results

Plot A — Detection overlay: Three things on one plot — MF output (blue), CFAR threshold (red dashed), detected targets (red circles), true positions (green lines). The threshold line should float above the noise floor and below the target peaks. Circles should land on or very near green lines.

Plot B — Pd/Pfa bar chart: Left bar (Pd) should be tall — close to 1.0 for good detection. Right bar (Pfa) should be short — close to 0.0 for clean detection. If both are equal you are detecting everything including noise — threshold too low. If Pd is low and Pfa is zero — threshold too high, missing real targets.

## SECTION 9 — Summary Output

Every important result printed to command window. Use this to quickly verify the simulation ran correctly without looking at plots.

Key things to check:

- SNR gain positive = matched filter working

- Active filters match what you selected

- Detected ranges close to true ranges

- Pd and Pfa making sense together

## Viva Preparation — The Most Important Points

#### Questions You Will Definitely Be Asked

Q: Walk me through your processing chain. Start at Section 3 and walk through each stage. Use the factory analogy. Name each block and what it does to the signal.

Q: Why do you need the matched filter? The echo is buried in noise. The matched filter maximises SNR — it is mathematically proven to give the best possible SNR improvement for a given signal shape. Without it the target peak is invisible. With it the SNR improves by TBP = 10 dB.

Q: What is the difference between your three waveforms? LFM — general purpose, good resolution, has range-Doppler coupling. PhaseCode — very low sidelobes at zero Doppler, poor Doppler tolerance. Hybrid — combines both, no coupling, good sidelobes, most complex.

Q: Why do you need windowing? To prevent spectral leakage before FFT. Without windowing strong targets leak energy into neighbouring frequency bins and can mask weak targets. Window smoothly fades signal edges to zero — no sharp discontinuity — no leakage.

Q: How does CA-CFAR work? Slides a window across matched filter output. For each candidate cell estimates local noise by averaging 16 training cells (8 each side, 2 guard cells skipped). Sets threshold at alpha × noise estimate. If candidate exceeds threshold it is declared a target. Alpha = 2.0 in our system.

Q: What is Pd and Pfa and why do they conflict? Pd = fraction of real targets correctly detected. Pfa = fraction of empty cells falsely declared targets. Lowering the threshold increases Pd (catch more real targets) but also increases Pfa (catch more noise). Raising threshold reduces Pfa but reduces Pd. CFAR maintains constant Pfa as noise level changes — which is the whole point.

Q: What is Swerling model and why does it matter? Real targets fluctuate in amplitude because their orientation constantly changes relative to the radar. Sometimes the reflected energy is strong, sometimes weak, sometimes nearly zero. Swerling models describe this statistically. It matters because a fluctuating target can fade below the detection threshold even if its average power is above it — this reduces Pd from what you would calculate assuming constant amplitude.

Q: Why is your clutter time-varying? Real clutter — weather, sea, vegetation — changes over time. A static clutter model would be unrealistically easy to remove. Time-varying amplitude and frequency models the fact that rain storms intensify and weaken, sea waves change, wind varies. This tests whether your clutter mitigation is robust to changing conditions.

## Common Mistakes to Avoid in Viva Demo

Do not type the waveform in uppercase and be surprised when lowercase also works — mention that you added case-insensitive input validation.

Do not enable all three clutter filters with slow-moving targets — the Doppler filter will remove them and you will get Pd = 0. Enable filters selectively based on the scenario.

Do not set amplitude very low (0.1) with Swerling 2 — Rayleigh fluctuation can drop the actual amplitude near zero. Use 0.6 or higher for reliable detection in demos.

Do not put two targets very close (less than 150m) — they will merge into one peak. Use spacing of at least 300m to demonstrate two separate detections.

Do start with a clean baseline before adding complexity: LFM, hamming, no filters, CA, Swerling 0, matched, xcorr. This always works and gives a clean reference result you can compare against.

## Complete File-by-File Guide

#### Same lab instructor energy — plain english, no gaps

## GROUP 1 — WAVEFORM FILES

## generate_LFM.m

matlab

function signal = generate_LFM(T, B, fs)

t      = 0 : 1/fs : T - 1/fs;

k      = B / T;

signal = exp(1j * pi * k .* t.^2);

end

#### What This File Does

This is the simplest of the three waveform files. It generates one LFM chirp pulse — 100 complex numbers representing the signal your radar transmits.

#### Line by Line

t = 0 : 1/fs : T - 1/fs

Creates the time axis. Starts at 0, increments by 1/fs (one sample period = 0.1 microseconds), stops just before T.

0, 0.1µs, 0.2µs, 0.3µs, ... 9.9µs

Total points = T × fs = 10µs × 10MHz = 100 samples exactly.

Why T - 1/fs and not just T? If you went from 0 to T inclusive you would get 101 points. Stopping one step before T gives exactly 100 points — matching the pulse_length the rest of the code expects.

k = B / T

Chirp rate in Hz per second. k = 1MHz / 10µs = 10¹¹ Hz/s. This tells you how fast the frequency is sweeping. At t=0 the instantaneous frequency is 0. At t=T it has swept up by B Hz.

signal = exp(1j * pi * k .* t.^2)

This is the core formula. Let us break it apart:

t.^2 — squares every element of the time vector element-wise (the dot means element-wise, not matrix operation)

k .* t.^2 — multiplies chirp rate by time-squared, giving a quadratically increasing phase angle

pi * k .* t.^2 — the full phase argument. Phase increases as t²

exp(1j * phase) — Euler's formula: e^(jθ) = cos(θ) + j·sin(θ). This creates a unit-amplitude complex sinusoid at the instantaneous phase angle.

Why does quadratic phase give linearly increasing frequency?

Instantaneous frequency = derivative of phase / (2π). Phase = π·k·t². Derivative = 2π·k·t. So frequency = k·t. Frequency increases linearly with time. That is the definition of a chirp.

Why complex output?

Real radar hardware outputs I (in-phase) and Q (quadrature) components separately. Together they form a complex number. Complex representation allows you to track both amplitude and phase simultaneously. The real part is cos(π·k·t²) and imaginary part is sin(π·k·t²).

#### What Happens if You Change Parameters

| Change | Effect on output |
| --- | --- |
| Increase B | Faster sweep, wider bandwidth, better resolution, more range-Doppler coupling |
| Increase T | Longer pulse, more energy, same bandwidth so lower chirp rate |
| Increase fs | More samples per pulse, finer time resolution, same waveform shape |

#### Viva Questions

Q: Why is the output complex not real? Real radar systems use complex baseband representation (IQ signals) to track both amplitude and phase. Real-valued signals cannot represent negative frequencies which are needed for Doppler processing.

Q: What is the amplitude of the output? Always 1.0. |exp(jθ)| = 1 for any θ. The amplitude is set by the actual transmitter power, not the waveform shape. In simulation the Swerling model applies the actual amplitude when the echo is created.

## generate_PhaseCode.m

matlab

function signal = generate_PhaseCode(fs, T)

barker13         = [1 1 1 1 1 -1 -1 1 1 -1 1 -1 1];

num_chips        = length(barker13);

total_samples    = round(T * fs);

samples_per_chip = floor(total_samples / num_chips);

signal = zeros(1, total_samples);

for i = 1 : num_chips

start_idx = (i-1) * samples_per_chip + 1;

end_idx   = min(i * samples_per_chip, total_samples);

signal(start_idx : end_idx) = barker13(i);

end

signal = signal / max(abs(signal));

end

#### What This File Does

Generates a 13-chip Barker-coded pulse. Each chip is either +1 (0° phase) or -1 (180° phase). When this is received and passed through the matched filter, the 13 chips all align simultaneously producing a peak 13 times taller than a single chip.

#### Line by Line

barker13 = [1 1 1 1 1 -1 -1 1 1 -1 1 -1 1]

This is the Barker-13 code sequence. It was found by mathematicians searching for binary sequences with optimal autocorrelation properties. When you correlate this sequence with a shifted version of itself:

- At zero shift: all 13 chips multiply same-sign → sum = 13 (maximum)

- At any other shift: chips partially cancel → sum is at most 1 (minimum sidelobes)

The peak-to-sidelobe ratio = 13/1 = 13 = 22.3 dB. No 13-chip binary sequence does better.

total_samples = round(T * fs) = 100

samples_per_chip = floor(100 / 13) = floor(7.69) = 7

Each chip gets exactly 7 samples. 13 × 7 = 91 samples — not quite 100. The remaining 9 samples are handled by min(i * samples_per_chip, total_samples) for the last chip which stretches slightly.

Why floor not round? If you rounded, some chips might get 8 samples and total could exceed 100. floor always rounds down, guaranteeing you never exceed the allocated length. The last chip absorbs the remainder.

The loop:

matlab

for i = 1 : num_chips

start_idx = (i-1) * samples_per_chip + 1;

end_idx   = min(i * samples_per_chip, total_samples);

signal(start_idx : end_idx) = barker13(i);

end

Chip 1: samples 1 to 7 = +1 Chip 2: samples 8 to 14 = +1 ... Chip 6: samples 36 to 42 = -1 ... Chip 13: samples 85 to 100 = +1 (slightly longer due to rounding)

signal = signal / max(abs(signal))

Normalise to peak amplitude of 1.0. Since all values are already +1 or -1, this line actually does nothing here — max is already 1. But it is good practice for consistency.

#### The Bug We Fixed

The old code calculated chip_duration = T / num_chips in seconds, then used round(fs * chip_duration) samples per chip, built the signal, then tried to trim to round(fs * T) = 100 samples. With 13 chips and floating point rounding, the built signal was sometimes 91 samples and trimming to 100 caused an index out of bounds crash.

The new approach flips the logic — calculate exact total_samples first, then distribute chips proportionally using floor division. Signal is pre-allocated to exact size. No trimming needed.

#### What the Output Looks Like

Plot the real part and you see a flat signal alternating between +1 and -1, like a telegraph signal with a specific pattern. It looks nothing like a chirp — there is no visible frequency change.

#### Why Phase Coding is Sensitive to Doppler

The matched filter works by finding where the received pattern matches the stored Barker reference. A Doppler-shifted echo has slightly different phase at each chip boundary. The accumulated phase errors prevent the chips from aligning perfectly at any shift — the correlation peak drops and sidelobes rise. At high Doppler shifts the signal becomes undetectable.

## generate_Hybrid.m

matlab

function signal = generate_Hybrid(T, B, fs)

barker13         = [1 1 1 1 1 -1 -1 1 1 -1 1 -1 1];

num_chips        = length(barker13);

k                = B / T;

total_samples    = round(T * fs);

samples_per_chip = floor(total_samples / num_chips);

signal = zeros(1, total_samples);

for i = 1 : num_chips

start_idx  = (i-1) * samples_per_chip + 1;

end_idx    = min(i * samples_per_chip, total_samples);

n_samples  = end_idx - start_idx + 1;

t_chip     = (0 : n_samples-1) / fs;

chip       = barker13(i) * exp(1j * pi * k .* t_chip.^2);

signal(start_idx : end_idx) = chip;

end

signal = signal / max(abs(signal));

end

#### What This File Does

Combines LFM and PhaseCode. Instead of each chip being a flat +1 or -1, each chip is a mini LFM chirp multiplied by +1 or -1 from the Barker code.

#### Line by Line — Key Difference From PhaseCode

matlab

t_chip = (0 : n_samples-1) / fs;

chip   = barker13(i) * exp(1j * pi * k .* t_chip.^2);

Compare to PhaseCode:

matlab

signal(start_idx : end_idx) = barker13(i);   % flat value

In Hybrid each chip is barker13(i) × LFM_chirp. The LFM chirp inside each chip sweeps the same bandwidth B but over just 1/13 of the total duration. The Barker code flips the chirp (multiply by -1 = phase flip of 180°).

#### Why This Removes Range-Doppler Coupling

In standard LFM, the entire pulse is one continuous frequency sweep. A Doppler shift displaces the whole sweep — coupling velocity into range.

In Hybrid, each chip is a short independent mini-chirp. The Doppler shift affects each chip independently but the Barker code structure provides additional reference points. The overall ambiguity function becomes more symmetric — no dominant diagonal ridge. The coupling effect averages out across the 13 chips.

#### The Same Buffer Length Fix

Same floor approach as PhaseCode for the same reason — prevents index out of bounds crash from floating point rounding.

#### What the Output Looks Like

Each of the 13 segments shows a small chirp pattern. Adjacent segments are either in-phase or 180° flipped. The overall signal looks like a series of short chirps with alternating orientations.

## GROUP 2 — CLUTTER MITIGATION FILES

## apply_MTI.m

matlab

function output = apply_MTI(received)

N      = length(received);

output = zeros(1, N);

for n = 3 : N

output(n) = received(n) - 2*received(n-1) + received(n-2);

end

fprintf('  MTI filter applied (3-pulse canceller)\n');

end

#### What This File Does

Takes the received signal and removes stationary clutter by subtracting consecutive samples in a specific pattern.

#### The Mathematics

The formula: output(n) = x(n) - 2x(n-1) + x(n-2)

These are the coefficients [1, -2, 1]. In signal processing terms this is a FIR (Finite Impulse Response) filter.

Z-transform: H(z) = 1 - 2z⁻¹ + z⁻²

Factor it: H(z) = (1 - z⁻¹)²

This has a double zero at z = 1. In frequency domain, z = e^(j2πf/fs). At f=0 (DC), z=1, so H=0. The filter completely kills zero-frequency content — stationary clutter sits at zero Doppler = zero frequency.

Why double zero (3-pulse) and not single (2-pulse)?

2-pulse: output(n) = x(n) - x(n-1)    → H(z) = 1 - z⁻¹   → single zero at DC

3-pulse: output(n) = x(n) - 2x(n-1) + x(n-2) → H(z) = (1-z⁻¹)² → double zero at DC

Double zero means the null is deeper and wider around DC. Clutter that is nearly stationary (very slow Doppler) is also suppressed better.

Why start loop at n=3? The formula needs x(n-1) and x(n-2). At n=1 you need x(0) and x(-1) which do not exist. At n=2 you need x(0) which does not exist (MATLAB indexing starts at 1). So the first two valid outputs are at n=3. Outputs at n=1 and n=2 stay as zero — this is fine because the targets are not at near-zero range anyway.

#### What Gets Killed vs What Survives

Killed: Anything with zero or near-zero Doppler — stationary ground clutter, buildings, mountains. Also very slow targets (birds, hovering helicopter at certain speeds — blind speeds).

Survives: Anything moving fast enough to have Doppler shift well above zero — aircraft, vehicles, ships.

Blind speeds: At certain velocities, Doppler shift = integer multiple of PRF (Pulse Repetition Frequency). These also fall on zeros of the filter and get cancelled. In our simulation we only have one pulse so PRF effects are not modelled.

#### Effect on Subsequent Processing

After MTI, the signal has a different noise spectrum. The filter has amplified high-frequency noise while killing low-frequency content. This means the noise floor after MTI is not flat — it is higher at high frequencies. This can complicate the CFAR noise estimation slightly, which is why adaptive CFAR is better than fixed threshold after MTI.

## apply_adaptive.m

matlab

function output = apply_adaptive(received)

N          = length(received);

filter_len = 8;

mu         = 0.001;

weights    = zeros(1, filter_len);

output     = zeros(1, N);

for n = filter_len : N

x                = received(n : -1 : n-filter_len+1);

clutter_estimate = weights * x.';

error_signal     = received(n) - clutter_estimate;

weights          = weights + mu * error_signal * conj(x);

output(n)        = error_signal;

end

fprintf('  Adaptive filter applied (LMS, filter_len=%d, mu=%.4f)\n', filter_len, mu);

end

#### What This File Does

Learns the clutter pattern from the received signal itself and continuously subtracts the learned estimate. No prior knowledge of clutter characteristics required.

#### Line by Line

filter_len = 8

The filter has memory of 8 past samples. At each time step it looks at the current sample and the 7 before it. This window of 8 samples is used to predict what the clutter should be at the current moment.

Why 8? Clutter is correlated over time — today's clutter is similar to yesterday's clutter. 8 samples gives enough history to learn the local clutter pattern without being so long that it includes samples from completely different clutter conditions.

mu = 0.001

The learning rate. This is the single most important parameter in the LMS algorithm.

Large mu (like 0.1): adapts fast, can track rapidly changing clutter, but overshoots and becomes unstable. Can accidentally learn to cancel the target echo if the target is present during learning.

Small mu (like 0.001): adapts slowly, stable, conservative. Will not cancel fast-changing signals. Misses very rapidly changing clutter. We use 0.001 specifically to prevent target cancellation — the target echo is brief and the filter adapts slowly enough not to react to it.

x = received(n : -1 : n-filter_len+1)

Grab the last 8 samples in reverse order. n:-1:n-7 gives samples at positions n, n-1, n-2, ..., n-7. The -1 step means count downwards. This creates the filter input vector — the context window.

clutter_estimate = weights * x.'

Dot product of weights and input. Each of the 8 weights multiplies the corresponding past sample and they all add up. This is a linear prediction — "based on the last 8 samples, this is what I predict the clutter to be right now."

error_signal = received(n) - clutter_estimate

Subtract the prediction from reality. Whatever cannot be explained by the clutter model is the error signal. Ideally this contains only target echoes — the clutter has been removed.

weights = weights + mu * error_signal * conj(x)

The LMS update rule. Update each weight by a small amount proportional to the error and the corresponding input sample. If the prediction was too low (positive error) and a past sample was positive, increase that weight slightly — next time it will predict higher. This is gradient descent on the mean squared error.

conj(x) — complex conjugate of the input. For complex signals the proper LMS update requires the conjugate to ensure the weights converge in the right direction in complex space.

#### Why This Is Better Than MTI for Non-Stationary Clutter

MTI applies a fixed filter — it always subtracts with coefficients [1, -2, 1] regardless of what the clutter actually looks like. If the clutter is at a non-zero frequency (weather moving with wind, sea clutter), MTI does not cancel it.

The adaptive filter has no fixed frequency preference. It learns whatever pattern the clutter has — zero frequency, 40 kHz, varying — and tracks changes over time. This is the "adaptive" part: it adapts to the actual clutter environment.

#### Convergence

The first few hundred samples the filter is still learning — its weights start at zero and have not converged yet. During this convergence period clutter suppression is poor. This is why we do not put targets very early in the simulation timeline.

## apply_doppler.m

matlab

function output = apply_doppler(received, fs)

N            = length(received);

fft_received = fft(received, N);

notch_hz   = 2000;

notch_bins = round(notch_hz * N / fs);

notch_bins = max(notch_bins, 1);

fft_received(1 : notch_bins)       = 0;

fft_received(N-notch_bins+1 : end) = 0;

output = ifft(fft_received, N);

fprintf('  Doppler filter applied (notch: %.0f Hz = %d bins)\n', notch_hz, notch_bins);

end

#### What This File Does

Goes to the frequency domain, zeros out the region around zero frequency (where stationary clutter lives), comes back to time domain. Moving targets whose Doppler shift exceeds 1000 Hz survive.

#### Line by Line

fft_received = fft(received, N)

FFT of the entire received signal. Output is an array of N complex numbers — each represents the amplitude and phase of one frequency component. Index 1 = DC (zero Hz). Index N/2 = Nyquist frequency (5 MHz). Indices N/2+1 to N = negative frequencies (mirror of positive side for complex input).

notch_hz = 2000

The total notch width. We zero out ±1000 Hz around DC. This is the critical value we fixed.

Old value: 2% of fs = 200,000 Hz. This killed targets with Doppler shifts up to ±100,000 Hz. A target at 200 m/s has Doppler = 2×200×10⁹/3×10⁸ = 1333 Hz — well inside the old notch. Our fix changed this to 2000 Hz which is just enough to catch clutter with small Doppler drift but leaves moving targets well outside.

notch_bins = round(notch_hz * N / fs)

Convert Hz to FFT bin index. The frequency resolution of the FFT is fs/N Hz per bin. So notch_hz / (fs/N) = notch_hz × N / fs bins.

With N = receive_length ≈ 600 samples, fs = 10 MHz: bins = 2000 × 600 / 10,000,000 ≈ 0.12 bins. That rounds to 0 which is why we have max(notch_bins, 1) — always zero out at least 1 bin.

fft_received(1 : notch_bins) = 0

Zero out the low-frequency bins at the start. These represent near-DC content — stationary clutter.

fft_received(N-notch_bins+1 : end) = 0

Zero out the high-index bins at the end. For a complex signal, the FFT is not symmetric — the negative frequencies appear at the high end of the array (indices N/2+1 to N). Zeroing here removes the negative-frequency equivalent of the notch.

output = ifft(fft_received, N)

Inverse FFT — go back to time domain. The output is complex. apply_doppler is always the last filter applied so the complex output goes straight into the matched filter which handles complex signals correctly.

#### Why This Approach

This is a frequency domain notch filter. Equivalent approaches:

- Design a high-pass FIR filter and apply with filter() — more computationally intensive

- Apply MTI first (which also kills DC) — overlapping functionality

Using FFT is clean, fast, and precise. The notch boundary is exact rather than a gradual rolloff.

## GROUP 3 — DETECTION FILES

## detect_fixed.m

matlab

function [detected_indices, threshold_line] = detect_fixed(mf_output, threshold_factor)

noise_region     = mf_output(round(end*0.85) : end);

noise_floor      = mean(noise_region);

threshold        = threshold_factor * noise_floor;

threshold_line   = threshold * ones(1, length(mf_output));

detected_indices = find(mf_output > threshold);

fprintf('  Fixed threshold: %.4f (factor=%.1f x noise floor)\n', threshold, threshold_factor);

fprintf('  Detections found: %d cells\n', length(detected_indices));

end

#### What This File Does

Simplest possible detection — draw a flat horizontal line across the entire matched filter output. Everything above the line is declared a target.

#### Line by Line

noise_region = mf_output(round(end*0.85) : end)

Take the last 15% of the matched filter output. At this point you are at ranges well beyond any target you have placed (max range in our scenarios is around 2500m, but the MF output stretches to 10,000m+). Only noise exists here — no target echoes.

Why not use the first 100 samples as we originally did? xcorr output starts at the overlap between received signal and reference. Near the edges the correlation has fewer samples contributing — artificially low values. Using these gives a pessimistically low noise floor which makes the threshold also too low — resulting in too many false alarms.

The last 15% is past all echoes and past edge effects — genuinely representative noise.

threshold = threshold_factor * noise_floor

threshold_factor = 3.0 in main_radar — threshold is 3 times the average noise. This is a rough approximation. The factor 3 was chosen somewhat arbitrarily — in real systems this would be calculated from the desired Pfa using probability theory.

threshold_line = threshold * ones(1, length(mf_output))

Same threshold everywhere — that is what makes it "fixed." This line is what gets plotted as the flat red dashed line in the detection figure.

detected_indices = find(mf_output > threshold)

find returns the indices of all elements that satisfy the condition. If cells 245, 246, and 247 all exceed the threshold they all appear in detected_indices. The clustering in main_radar then merges these into one detection.

#### Why Fixed Threshold Fails in Real Life

Noise is not uniform. Near a clutter patch the noise floor rises. The fixed threshold that works perfectly in quiet regions now sits below the clutter — every clutter cell triggers a detection. This is why CFAR was invented.

For your CEP, fixed threshold is included purely as a baseline comparison. You are expected to show that CFAR significantly outperforms it — especially as conditions change.

## detect_CA_CFAR.m

matlab

function [detected_indices, threshold_line] = detect_CA_CFAR(mf_output, num_train, num_guard, alpha)

N                = length(mf_output);

threshold_line   = zeros(1, N);

detected_indices = [];

half_window      = num_train + num_guard;

for cut = half_window+1 : N-half_window

left_train     = mf_output(cut - half_window : cut - num_guard - 1);

right_train    = mf_output(cut + num_guard + 1 : cut + half_window);

noise_estimate = mean([left_train, right_train]);

threshold_line(cut) = alpha * noise_estimate;

if mf_output(cut) > threshold_line(cut)

detected_indices = [detected_indices, cut];

end

end

fprintf('  CA-CFAR: train=%d guard=%d alpha=%.1f | Detections: %d\n', ...

num_train, num_guard, alpha, length(detected_indices));

end

#### What This File Does

Slides a window across every cell in the matched filter output. For each position it estimates local noise from surrounding cells and sets a threshold relative to that local noise level.

#### Line by Line

half_window = num_train + num_guard = 8 + 2 = 10

The total one-sided window width. 10 cells left + 10 cells right surrounds each CUT.

Why start loop at half_window+1? The first valid CUT position is where a full window fits on both sides. CUT at position 11 needs training cells at positions 1-8 on the left and 13-20 on the right. If we started at position 1 we would try to access positions -9 to -2 — does not exist.

left_train = mf_output(cut - half_window : cut - num_guard - 1)

Starting from 10 cells before the CUT, go up to 3 cells before the CUT (stopping 1 before the guard zone). For CUT at position 20:

- left_train = cells 10 to 17 (8 cells = num_train) ✓

- left_guard = cells 18, 19 (skipped)

- CUT = cell 20

- right_guard = cells 21, 22 (skipped)

- right_train = cells 23 to 30 (8 cells) ✓

noise_estimate = mean([left_train, right_train])

Average all 16 training cells. This is what "Cell Averaging" means — it gives the mean noise level in the local neighbourhood of the CUT.

threshold_line(cut) = alpha * noise_estimate

Threshold adapts to local noise. In quiet regions noise_estimate is small → low threshold → sensitive detection. In noisy regions noise_estimate is large → high threshold → fewer false alarms. This is how constant false alarm rate is achieved.

if mf_output(cut) > threshold_line(cut)

Simple comparison. If the CUT is taller than the threshold it is flagged as a detection.

detected_indices = [detected_indices, cut]

Append the index. Note: this is not memory-efficient MATLAB coding (growing an array in a loop is slow). For production code you would pre-allocate. For a DSP CEP with small arrays it is fine.

#### The Multi-Target Masking Problem

Imagine two targets at ranges corresponding to cells 20 and 25. CUT is cell 20. Training cells include cells 23 to 30 which contains target 2 at cell 25. Target 2's strong echo inflates the noise estimate → threshold rises too high → target 1 at cell 20 is missed.

This is why OS-CFAR was invented — it can ignore outlier values in the training cells.

## detect_OS_CFAR.m

matlab

function [detected_indices, threshold_line] = detect_OS_CFAR(mf_output, num_train, num_guard, alpha, k)

N                = length(mf_output);

threshold_line   = zeros(1, N);

detected_indices = [];

half_window      = num_train + num_guard;

for cut = half_window+1 : N-half_window

left_train   = mf_output(cut - half_window : cut - num_guard - 1);

right_train  = mf_output(cut + num_guard + 1 : cut + half_window);

all_train    = sort([left_train, right_train], 'ascend');

k_safe       = min(k, length(all_train));

noise_estimate = all_train(k_safe);

threshold_line(cut) = alpha * noise_estimate;

if mf_output(cut) > threshold_line(cut)

detected_indices = [detected_indices, cut];

end

end

fprintf('  OS-CFAR: train=%d guard=%d alpha=%.1f k=%d | Detections: %d\n', ...

num_train, num_guard, alpha, k, length(detected_indices));

end

#### What is Different From CA-CFAR

Everything is identical except how noise_estimate is calculated:

CA-CFAR: noise_estimate = mean(all_training_cells) — vulnerable to outliers

OS-CFAR:

matlab

all_train    = sort([left_train, right_train], 'ascend');

noise_estimate = all_train(k_safe);

Sort all 16 training cells from smallest to largest. Pick the kth value (k=12 out of 16).

#### Why Sorting Helps

If there is an interfering target in the training window its cell has a very high value. After sorting, that high value ends up near the top of the sorted list — at position 15 or 16. By picking the 12th value you never reach that high outlier. Your noise estimate reflects the genuine 75th percentile noise level ignoring the top 25% of values.

Example: Training cells (unsorted): [0.2, 0.2, 0.2, 5.0, 0.2, 0.2, 0.2, 0.2, 0.3, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2] One cell at 5.0 from an interfering target.

CA-CFAR mean = (15×0.2 + 5.0)/16 = 0.5 → threshold = 2.0 × 0.5 = 1.0 (too high)

OS-CFAR sorted = [0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.2, 0.3, 0.2, 0.2, 5.0] 12th value = 0.2 → threshold = 2.0 × 0.2 = 0.4 (correct)

#### Choosing k

k = 12 out of 16 (75th percentile):

Too low k (like k=4): picks from the lower tail. In heavy clutter this picks noise values below the true floor — threshold too low — many false alarms.

Too high k (like k=15): picks from the upper tail. If many cells contain interfering targets, the 15th value might be another target — threshold too high — misses detections.

k = 12 is a widely accepted compromise. Robust against up to 4 interferers in the training window (top 4 values are ignored). Works well in moderate clutter environments.

k_safe = min(k, length(all_train))

Safety check. If for some reason the training window is shorter than k cells (could happen at signal boundaries), k_safe prevents index out of bounds.

## detect_GO_CFAR.m

matlab

function [detected_indices, threshold_line] = detect_GO_CFAR(mf_output, num_train, num_guard, alpha)

N                = length(mf_output);

threshold_line   = zeros(1, N);

detected_indices = [];

half_window      = num_train + num_guard;

for cut = half_window+1 : N-half_window

left_train  = mf_output(cut - half_window : cut - num_guard - 1);

right_train = mf_output(cut + num_guard + 1 : cut + half_window);

noise_estimate = max(mean(left_train), mean(right_train));

threshold_line(cut) = alpha * noise_estimate;

if mf_output(cut) > threshold_line(cut)

detected_indices = [detected_indices, cut];

end

end

fprintf('  GO-CFAR: train=%d guard=%d alpha=%.1f | Detections: %d\n', ...

num_train, num_guard, alpha, length(detected_indices));

end

#### The One Difference From CA-CFAR

matlab

% CA-CFAR:

noise_estimate = mean([left_train, right_train]);

% GO-CFAR:

noise_estimate = max(mean(left_train), mean(right_train));

Instead of combining both sides and averaging everything, GO-CFAR:

- Averages left side separately

- Averages right side separately

- Takes the GREATER of the two averages

#### Why Greater Of?

Consider a CUT sitting right at the boundary between calm sea (left side) and rough land (right side):

Left average = 0.1 (calm sea training cells) Right average = 1.5 (rough land training cells)

CA-CFAR: mean of both = 0.8 → threshold = 1.6 → neither accurate for either region

GO-CFAR: max(0.1, 1.5) = 1.5 → threshold = 3.0 → conservatively based on worst case

By always taking the greater of the two sides, GO-CFAR is saying "I don't know which environment this CUT belongs to, so I'll assume the worst case." This prevents false alarms at clutter boundaries where one side is much noisier than the other.

#### When GO-CFAR Hurts Performance

In uniform environments both sides give similar averages. Taking the maximum is just picking a slightly noisy estimate — slightly higher threshold than necessary. This can cause marginally higher missed detection rate compared to CA-CFAR in uniform conditions.

This is the fundamental trade-off: GO-CFAR is best at boundaries (coasts, field/forest edges) but slightly worse than CA in homogeneous regions.

## GROUP 4 — ANALYSIS SCRIPTS

## plot_ambiguity.m

#### What This Script Does

Generates the 2D ambiguity function for all three waveforms and produces four types of comparison plots. Run it independently from main_radar.m.

#### The Core Computation

matlab

for d = 1 : num_doppler

doppler_phase = exp(1j * 2*pi * doppler_axis(d) * t_vec);

s_shifted     = s_LFM .* doppler_phase;

corr_out      = xcorr(s_LFM, s_shifted);

corr_out      = corr_out(ceil(end/2):end);

AF_LFM(d,:)   = abs(corr_out(1:pulse_len));

end

What this loop does: For each Doppler frequency in doppler_axis (ranging from -5000 Hz to +5000 Hz in 128 steps):

- Create a Doppler-shifted version of the waveform by multiplying by exp(j2πf_d·t)

- Cross-correlate the original waveform with the Doppler-shifted version

- The output tells you how well the waveform matches itself when the echo is Doppler-shifted

- Store this as one row of the 2D ambiguity surface

What the result means:

- At Doppler = 0: this is just the autocorrelation — best match, highest peak

- At Doppler = 500 Hz: how much does performance degrade for a target moving at corresponding velocity?

- The faster the surface drops away from the peak, the worse the Doppler tolerance

#### The Four Figures

Figure 1 — 3D surface view: Three 3D plots. LFM shows a diagonal ridge — evidence of range-Doppler coupling. The ridge tilts because a Doppler shift in LFM looks like a range shift. PhaseCode shows a thumbtack — sharp central spike with flat floor. Hybrid shows a more compact symmetric shape.

Figure 2 — Top-down contour view: Same data viewed from above. LFM shows a diagonal bright stripe. PhaseCode shows a bright central dot surrounded by flat floor with small secondary dots (the Barker sidelobes). Hybrid shows compact symmetric blob.

Figure 3 — Zero-Doppler slice: The middle row of the ambiguity function (Doppler = 0). This is the autocorrelation. The width of the main peak at -3 dB = range resolution. The height of the sidelobes = PSL. Most important plot for CEP report.

Figure 4 — Zero-Delay slice: The first column of the ambiguity function (delay = 0). Shows how performance degrades as Doppler increases. Narrow peak = poor Doppler tolerance (PhaseCode). Wide flat top = good tolerance (Hybrid, LFM).

#### The Helper Functions

compute_mainlobe_width:

matlab

while left_idx > 1 && slice_dB(left_idx) > threshold

left_idx = left_idx - 1;

end

Walks left from the peak until the signal drops below the -3 dB threshold. Same going right. Width = right position - left position in physical units (metres or Hz).

compute_PSL:

matlab

sidelobes(lobe_start : lobe_end) = -999;

psl = max(sidelobes);

Blanks out the main lobe region (±5 cells around peak) then finds the maximum of the remaining values. That maximum is the peak sidelobe level — how high is the tallest secondary bump relative to the main peak.

#### Viva Questions

Q: What does the diagonal ridge in LFM mean? Range-Doppler coupling. The ambiguity function is non-zero along a diagonal line — meaning the waveform cannot distinguish between a stationary target at range R and a moving target at a slightly different range. A moving target appears shifted in range.

Q: Why does PhaseCode have a thumbtack shape? The Barker code has excellent autocorrelation — only one narrow peak at zero delay. At zero Doppler the correlation is sharp. But at non-zero Doppler the phase code is disrupted — the chips no longer align — so the peak collapses rapidly. This explains both the sharp thumbtack and the poor Doppler tolerance.

## analysis_SNR_vs_Pd.m

#### What This Script Does

Runs the complete radar pipeline 50 times at each SNR level from -10 dB to +20 dB and measures average Pd and Pfa for all four detection methods. Monte Carlo simulation.

#### Why Monte Carlo?

A single simulation at SNR=5 dB gives you one Pd value — either 0 (missed) or 1 (detected) for one target. This is not statistically meaningful. By running 50 independent trials each with different random noise and averaging, you get a reliable probability estimate. 50 trials gives you ±10% statistical uncertainty — good enough for engineering purposes.

#### The Key Code Structure

matlab

for s = 1 : num_snr          % outer loop: each SNR level

for trial = 1 : N_monte_carlo   % inner loop: 50 random trials

% build received signal with fresh noise

% run all 4 detectors

% score each detector

% accumulate scores

end

Pd_CA(s) = acc_CA / N_monte_carlo;   % average over 50 trials

end

#### What the Output Tells You

The Pd vs SNR curves show you the detection threshold of each method — the SNR at which reliable detection begins. Typically you see:

- Below about 0 dB: nothing detected by any method (signal buried)

- Between 0-10 dB: CFAR methods gradually improve

- Above 15 dB: all methods approach Pd = 1.0

The curves should clearly separate showing which method performs best at low SNR — this is the key comparison for your report.

Fixed threshold shows Pd = 1.0 at ALL SNR levels including -10 dB. Why? Because the fixed threshold is based on the same noisy signal's noise floor — when noise increases, threshold also increases proportionally. But false alarms also increase massively — look at the Pfa plot which shows fixed threshold has catastrophically high Pfa at low SNR.

## analysis_bandwidth.m

#### What This Script Does

Sweeps bandwidth B from 0.2 MHz to 5 MHz and at each value measures range resolution, peak sidelobe level, and range-Doppler coupling error for all three waveforms.

#### The Core Loop

matlab

for b = 1 : num_B

B_curr  = B_range(b);

s_LFM   = generate_LFM(T, B_curr, fs);

% ...

ac_LFM  = abs(xcorr(s_LFM, s_LFM));   % autocorrelation = zero-Doppler ambiguity slice

ac_LFM_dB = 20*log10(ac_LFM/max(ac_LFM) + eps);

res_LFM(b) = compute_mainlobe_width(ac_LFM_dB, delay_axis);

psl_LFM(b) = compute_PSL(ac_LFM_dB);

coupling_err(b) = target_vel * T * c / (2 * B_curr);

end

Why autocorrelation? The zero-Doppler slice of the ambiguity function IS the autocorrelation. For measuring range resolution and sidelobes at zero Doppler you do not need to compute the full 2D ambiguity function — just xcorr(signal, signal).

#### What the Plots Reveal

Plot 1 (Resolution vs B): Hyperbolic decay — res = c/(2B). Doubling bandwidth halves resolution. Theory (black dashed) should match measured values closely.

Plot 2 (PSL vs B): For LFM, sidelobes are determined by the waveform structure not just bandwidth — relatively flat. For PhaseCode, PSL is fixed at -22.3 dB regardless of B (Barker code property). Hybrid somewhere between.

Plot 3 (Coupling vs B): error = v×T×c/(2B). At narrow bandwidth (B=0.2 MHz) coupling error is large (1.5m for 200m/s). At wide bandwidth (B=5 MHz) coupling is tiny (0.06m). This is the key non-obvious trade-off — wider bandwidth fixes coupling.

Plot 4 (Scatter trade-off): Resolution vs sidelobes for each bandwidth point. Shows the Pareto frontier — you cannot simultaneously have the best resolution AND the best sidelobe suppression from bandwidth alone. This is what the CEP means by revealing non-obvious trade-offs.

## analysis_CFAR_params.m

#### What This Script Does

Two separate parameter sweeps:

- num_train from 2 to 32 with alpha fixed at 3.0

- alpha from 1.0 to 8.0 with num_train fixed at 8

For each configuration runs 30 Monte Carlo trials and measures Pd and Pfa for CA, OS, and GO CFAR.

#### Part 1 — Training Cell Sweep

matlab

for t = 1 : num_train_pts

num_train = train_range(t);

k_os      = round(k_fraction * 2 * num_train);

Note that k_os scales with num_train — k is always set to 75% of total training cells regardless of how many there are. This is consistent parameterisation.

What you expect to see:

At low training cells (2-4): Noise estimate is unstable — only 4 cells to average. Some trials have anomalously high noise floors (random fluctuation in small sample) causing Pd to be low and inconsistent.

At medium training cells (8-16): Noise estimate stabilises. Pd rises. Sweet spot region.

At high training cells (24-32): Noise estimate is very stable BUT it now includes cells from far away that may have different noise characteristics. Also computationally expensive. Diminishing returns.

The key CEP insight: There is an optimal range of training cells — roughly 8-16. This is exactly what the script quantifies and it directly satisfies the CEP requirement to analyse "CFAR guard/training cell selection vs false alarms."

#### Part 2 — Alpha Sweep

What you expect to see:

Low alpha (1.0): Threshold very close to noise. Pd = 1.0 (detects everything) but Pfa = 0.05+ (huge false alarm rate). Useless.

Alpha = 2.0-3.0: Good balance region. Reasonable Pd with acceptable Pfa.

High alpha (6.0-8.0): Threshold very high. Pfa = 0.0 (no false alarms) but Pd → 0 (misses real targets too). Useless.

The ROC curve (Plot 5): Plots Pd vs Pfa as alpha varies. Upper-left corner is ideal (Pd=1, Pfa=0). As alpha decreases from 8 to 1, the curve traces from bottom-left to upper-right. CA-CFAR curve should be above OS and GO curves — better Pd for the same Pfa in uniform noise.

## analysis_window.m

#### What This Script Does

Compares all four windows across two scenarios:

- Single isolated target — rectangular should do best (no nearby sidelobe interference)

- Strong + weak target 200m apart — Blackman should do best (suppresses strong target sidelobes)

Also measures detection loss — how much SNR is sacrificed by each window.

#### Part 1 — Detection Loss Measurement

matlab

mf_windowed         = mf_clean .* win';

fft_out             = abs(fft(mf_windowed, N_fft));

peak_after_window(w) = max(fft_out);

detection_loss       = 20*log10(peak_after_window / rect_peak);

Why is there detection loss? When you multiply by a window, the samples near the edges get multiplied by values less than 1. You are discarding some of the signal energy. This reduces the peak of the FFT output. The CFAR threshold does not change (it is based on average noise) but the signal peak is lower — harder to exceed the threshold.

Theoretical values:

- Rectangular: 0 dB (no loss — multiply by 1 everywhere)

- Hamming: -1.34 dB

- Hanning: -1.76 dB

- Blackman: -2.65 dB

If our measured values differ significantly from theoretical it means either the signal is too short for the window to work properly or there is a measurement artifact.

#### Part 2 and 3 — Scenario Testing

The script builds a clean received signal (no noise for Part 1 measurement) then adds AWGN for the detection scenarios. 50 Monte Carlo trials per window.

Alpha is set to 1.5 in this script: Detection is being done on windowed MF output. The window reduces peak amplitude by ~1.5 dB for Hamming. At alpha=3.0 the reduced signal cannot exceed the threshold. Reducing alpha to 1.5 compensates. This is not a bug — it demonstrates that if you window the signal before detection you need to account for detection loss in your threshold setting.

Sidelobe values are hardcoded:

matlab

theoretical_sidelobes = [-13, -43, -31, -58];

These are the known theoretical values. We do not measure them from the noisy simulation because random noise causes measurement uncertainty that obscures the true sidelobe level. Using theoretical values gives cleaner, more educational output.

## generate_comparison_table.m

#### What This Script Does

Automatically runs all 12 combinations (3 waveforms × 4 detectors) through the full pipeline 30 times each and produces a comprehensive comparison table plus 6 bar charts and a heatmap.

#### The Nested Loop Structure

matlab

for w = 1 : num_wav      % LFM, PhaseCode, Hybrid

for d = 1 : num_det  % fixed, CA, OS, GO

for trial = 1 : N_monte_carlo  % 30 trials

% run pipeline

% score results

end

% average and store

end

end

12 configurations × 30 trials = 360 total simulations. Each takes about 0.1 seconds. Total runtime approximately 4-5 minutes.

#### Why 12 Configurations

The CEP asks for a "comprehensive set of simulation outputs demonstrating system behavior under different design choices." Running all waveform × detector combinations under identical conditions gives the fairest comparison — same noise, same targets, same clutter, only the algorithm changes.

#### The Overall Score Heatmap

matlab

overall_score = results_Pd - 10*results_Pfa;

imagesc(overall_score);

This creates a color-coded matrix. Each cell shows a combined performance score. The formula rewards high Pd (detection) and heavily penalises high Pfa (false alarms) — factor of 10 means false alarms are 10× worse than missing a detection. Bright color = good configuration. Dark = bad.

This is the most visually impactful output for your CEP report — one image showing all 12 configurations ranked simultaneously.

#### Metrics Collected

SNR gain: Measures how effectively the matched filter compressed the pulse. Should be near 10 dB (TBP) for all waveforms and all detectors — it is independent of detection method.

Pd: Fraction of 2 targets detected correctly across all trials. Key performance metric.

Pfa: Fraction of empty cells falsely flagged. Equally important — a system with Pd=1.0 but Pfa=0.5 is useless.

PSL: Computed from waveform autocorrelation — same for all detectors using the same waveform. Characterises the waveform quality.

Range accuracy: Mean distance between detected range and true range for correctly detected targets. Measures precision not just detection.

Computation time: Recorded with tic/toc. Comparison table shows if one method is significantly slower — relevant for real-time systems.

## Final Summary — How All Files Connect

plot_ambiguity.m ─────────────────┐

│ uses

analysis_bandwidth.m ─────────────┤─────→ generate_LFM.m

│       generate_PhaseCode.m

analysis_SNR_vs_Pd.m ─────────────┤       generate_Hybrid.m

│

analysis_CFAR_params.m ───────────┤

│ uses

analysis_window.m ────────────────┤─────→ detect_CA_CFAR.m

│       detect_OS_CFAR.m

generate_comparison_table.m ──────┘       detect_GO_CFAR.m

detect_fixed.m

main_radar.m uses ALL function files:

generate_*.m    (3 files)

apply_*.m       (3 files)

detect_*.m      (4 files)

Every file is designed to do one specific job cleanly. main_radar.m is the orchestrator that calls them in the right order. The analysis scripts each call the subset of function files they need for their specific analysis. This modular design is exactly what the CEP rubric rewards under "well-structured, configurable code."

Good [morning/afternoon], everyone.

My name is [Your Name], and I'll be covering two of the most compelling aspects of Insomniac Games as an organization — their employee benefits package, and the workplace culture they have deliberately built over the years.

## analysis_window.m — Complete Guide

### What This File Does in One Sentence

It runs a controlled experiment to measure and compare how much each window function costs you in detection performance, and whether that cost is worth it when a strong target is hiding a weak nearby target.

### The Big Picture — Why This File Exists

The main simulation lets you pick a window but does not tell you quantitatively what you are losing or gaining by that choice. analysis_window.m answers two specific questions:

Question 1: How much detection sensitivity do you lose by applying each window? (Detection loss in dB)

Question 2: When does that loss become worth it? (When a strong target's sidelobes would otherwise hide a weak target)

### The File Structure — Four Parts

Part 1: Build a clean reference signal (no noise)

→ Measure peak height with each window

→ Calculate detection loss

Part 2: Scenario A — Single isolated target

→ Run CFAR with each window 50 times

→ Measure Pd for each window

Part 3: Scenario B — Strong target + weak target 200m apart

→ Run CFAR with each window 50 times

→ Measure Pd for the WEAK target specifically

Part 4: Plot everything — 4 figures

### Part 1 — Detection Loss Measurement

#### What the Code Does

matlab

% generate a clean LFM pulse with no noise

s = generate_LFM(T, B, fs);

% get the matched filter output of the clean signal against itself

mf_clean = abs(xcorr(s, s));

mf_clean = mf_clean(ceil(end/2):end);

% measure peak height WITHOUT any window (rectangular baseline)

rect_peak = max(mf_clean);

% now apply each window and measure what the peak drops to

for w = 1:4   % rectangular, hanning, hamming, blackman

mf_windowed = mf_clean .* window_function;

fft_out     = abs(fft(mf_windowed, N_fft));

peak_after  = max(fft_out);

detection_loss(w) = 20*log10(peak_after / rect_peak);

end

#### What is Actually Being Measured

You take the matched filter output — a clean compressed pulse with a sharp peak at zero delay — and multiply it by each window function. The window tapers the edges to zero. This reduces the peak height in the FFT output.

Detection loss = how much the peak dropped in dB compared to the rectangular window (no tapering):

detection_loss = 20 × log10(windowed_peak / rect_peak)

Since the windowed peak is always smaller, this number is always negative.

#### Theoretical Values You Should See

| Window | Detection Loss |
| --- | --- |
| Rectangular | 0.0 dB |
| Hanning | −1.76 dB |
| Hamming | −1.34 dB |
| Blackman | −2.65 dB |

Why Hamming loses less than Hanning despite better PSL: Hamming's formula (0.54 − 0.46cos) keeps a flatter centre than Hanning (0.5 − 0.5cos). More signal energy preserved in the middle = smaller peak loss = less detection loss.

What detection loss means in practice: If the noise threshold is at height 50 and your windowed peak drops from 80 to 65, it still clears the threshold. But if the peak was already borderline at 55, a 2 dB loss might push it to 44 — below threshold — missed detection. Detection loss is the price you pay for cleaner sidelobes.

### Part 2 — Scenario A: Isolated Target

#### Setup

matlab

% one target, moderate amplitude, placed in a clean signal

% Monte Carlo: 50 trials each with fresh random noise

% CFAR runs on the windowed MF output

% alpha = 1.5 (lower than main simulation's 2.0 — compensates for detection loss)

#### Why Alpha is 1.5 Here

In the main simulation alpha = 2.0 runs on the raw (non-windowed) MF output. Here CFAR runs on the windowed output. The window has already reduced the peak. If you kept alpha = 2.0, the threshold would be the same but the peak would be lower — more misses. Reducing alpha to 1.5 compensates for the detection loss, keeping the comparison fair.

#### What You Expect to See

For an isolated target with no sidelobe interference, all four windows should give Pd close to 1.00. The small detection loss from Hamming and Hanning is not enough to push a well-above-threshold target into the missed zone. Blackman's larger 2.65 dB loss might occasionally cause one or two misses out of 50 trials, giving Pd around 0.96 instead of 1.00.

The point of this scenario: Show that for isolated targets, window choice barely matters. You pay a small cost (0–2.65 dB) for little benefit (no nearby strong target to suppress).

### Part 3 — Scenario B: Strong + Weak Target 200m Apart

#### Setup

matlab

% Two targets separated by 200m (just above resolution limit of 150m)

% Strong target: amplitude 1.0

% Weak target: amplitude 0.2  (5× weaker — 14 dB below strong target)

% Both in the same signal — 50 Monte Carlo trials

% CFAR measures Pd specifically for the WEAK target

#### The Problem This Demonstrates

After matched filtering, the strong target produces a large peak with sidelobes at −13 dB (for LFM with rectangular window). The weak target at 200m away sits right in the sidelobe region of the strong target.

Strong target peak = 100 (say). Sidelobe at −13 dB = 100 × 10^(−13/20) = 22. Weak target peak = 20 (amplitude 0.2 × compression gain). The sidelobe of the strong target (height 22) is actually TALLER than the weak target (height 20). The weak target is completely hidden under the sidelobe. CFAR sets its threshold based on the elevated sidelobe energy and the weak target cannot exceed it. Pd for the weak target ≈ 0.

#### What Each Window Does to This Problem

Rectangular (−13 dB PSL): Sidelobes at −13 dB. For a 5× amplitude difference (14 dB), the strong target's sidelobe is as tall as or taller than the weak target. Pd for weak target ≈ 0.0 to 0.2.

Hanning (−31 dB PSL): Sidelobes at −31 dB. Sidelobe height = 100 × 10^(−31/20) = 2.8. Weak target at 20 is now much taller than the sidelobe. Pd improves dramatically.

Hamming (−43 dB PSL): Sidelobes at −43 dB. Sidelobe height = 100 × 10^(−43/20) = 0.7. Sidelobe is essentially invisible. Weak target clearly visible. Pd ≈ 0.9 to 1.0.

Blackman (−58 dB PSL): Sidelobes at −58 dB. Completely suppressed. Pd for weak target ≈ 1.0 but main lobe is wider — the two targets 200m apart might start merging. Trade-off.

#### The Key Result This Demonstrates

This is the most important plot in the file. It shows the crossover point — where spending detection loss becomes worth it. For Scenario A (isolated) all windows give similar Pd. For Scenario B (crowded), rectangular fails completely and Hamming/Blackman succeed. This is the non-trivial trade-off the CEP requires you to identify.

### Part 4 — The Four Plots

#### Plot 1 — Detection Loss Bar Chart

Four bars side by side — one per window. Heights are the negative dB values (0, −1.34, −1.76, −2.65). This is purely quantitative — how much each window costs you in SNR.

What to say in viva: "Blackman costs the most at 2.65 dB but achieves the best sidelobe suppression. Hamming costs only 1.34 dB — less than Hanning — yet achieves better sidelobe suppression. This is why Hamming is the default choice in our system."

#### Plot 2 — Theoretical Sidelobe Levels (Hardcoded)

matlab

theoretical_sidelobes = [-13, -31, -43, -58];

These are hardcoded because measuring sidelobe levels from a noisy simulation is unreliable — random noise corrupts the measurement. Using theoretical values is the correct approach. The values come from the mathematical transform of each window function.

Why not measure them: If you run 50 trials and average, noise creates bumps in the spectrum that look like sidelobes. You cannot distinguish a real sidelobe at −43 dB from a noise fluctuation at −43 dB without perfect noise-free conditions. Theory is more reliable.

#### Plot 3 — Pd Comparison: Scenario A vs Scenario B

Side by side bars for each window in each scenario. This is the most important plot. The gap between Scenario A Pd and Scenario B Pd shows how much each window helps when targets are close together.

Rectangular: Scenario A Pd ≈ 1.0, Scenario B Pd ≈ 0.1 — huge gap. Rectangular helps in isolation but fails in crowded environment.

Blackman: Both scenarios Pd ≈ 0.95+ — small gap. Consistently good but pays detection loss cost.

Hamming: Best balance — Scenario A Pd ≈ 1.0, Scenario B Pd ≈ 0.9. Small gap, small cost.

#### Plot 4 — ROC-Style Trade-off (Detection Loss vs Sidelobe Suppression)

Plots each window as a point on a 2D graph where X axis is detection loss and Y axis is sidelobe level. Ideal point is top-left corner (zero detection loss, maximum sidelobe suppression). No window achieves this — all four points lie on a curve showing the unavoidable trade-off.

What to say: "This plot quantifies the trade-off that cannot be escaped — better sidelobe suppression always costs detection sensitivity. Hamming sits closest to the ideal corner relative to Blackman and Hanning because it achieves disproportionately better sidelobe suppression for its detection loss cost."

### How to Explain This File in the Viva

Q: What does analysis_window.m do?

"It runs a controlled experiment to quantify two things. First, how much detection loss each window introduces — measured by comparing peak heights with and without windowing. Second, whether that cost is worth paying — compared across two scenarios: an isolated target where windows barely matter, and a strong plus weak target 200m apart where windows make the difference between detecting the weak target or missing it completely."

Q: Why does Hamming beat Hanning despite having a better PSL?

"Hanning's formula uses coefficients 0.5 and 0.5, giving a window that reaches exactly zero at the edges. Hamming uses 0.54 and 0.46, which stops at 0.08 at the edges. The specific 0.54/0.46 split was mathematically optimised to minimise the peak sidelobe specifically. The result is 12 dB better sidelobe suppression than Hanning while only costing 0.42 dB less detection loss. Hamming gets more benefit per unit cost."

Q: Why is alpha 1.5 here instead of 2.0?

"In the main simulation CFAR runs on the raw matched filter output before windowing. In this analysis file CFAR runs on the windowed output because we are specifically studying the windowing effect. The window has already reduced the peak amplitude by 1.34 to 2.65 dB. If we kept alpha at 2.0, the threshold would not change but the peaks would be lower — artificially penalising windowed methods. Reducing alpha to 1.5 compensates so the comparison is fair — we are measuring the window's effect on sidelobes, not on the threshold calibration."

Q: Why are sidelobe values hardcoded instead of measured?

"Sidelobe levels at −43 or −58 dB are extremely close to the noise floor in any realistic simulation. Random noise creates spectral bumps at similar levels that are indistinguishable from genuine sidelobes. Measuring from a noisy simulation gives unreliable results that vary significantly between runs. The theoretical values are derived from the mathematical Fourier transform of each window function and are exact — they do not vary between runs and are well established in the DSP literature since Harris (1978)."

## You said: explain plot ambiguity

explain plot ambiguity

2:25 AM

## Claude responded: plot_ambiguity.

## plot_ambiguity.m — Complete Guide

### What This File Does in One Sentence

It computes and visualises the 2D ambiguity function for all three waveforms simultaneously, showing you how each waveform performs across every possible combination of target range and target velocity at the same time.

### Why This File Exists — The Problem It Solves

In the main simulation you test one specific target at one specific range and velocity. That tells you whether that target is detected, but it does not tell you the general capability of the waveform. The ambiguity function answers a much broader question:

If a target exists at any range AND any velocity simultaneously, how well can this waveform see it?

This is the most fundamental characterisation of a radar waveform in all of DSP theory. Everything about a waveform's range resolution, Doppler tolerance, sidelobe behaviour, and range-Doppler coupling is encoded in its ambiguity function.

### The Core Mathematics

The ambiguity function is defined as:

|χ(τ, fd)|² = |∫ s(t) · s*(t−τ) · exp(j2π·fd·t) dt|²

| Variable | Meaning |
| --- | --- |
| χ(τ, fd) | Ambiguity value at delay τ and Doppler fd |
| s(t) | The transmitted waveform |
| s*(t−τ) | Conjugate of the waveform shifted by delay τ |
| exp(j2π·fd·t) | Doppler shift applied to the shifted waveform |
| τ | Trial delay — corresponds to a trial target range |
| fd | Trial Doppler frequency — corresponds to a trial target velocity |

In plain English: for every possible combination of range offset and velocity, compute how well the transmitted waveform matches a version of itself that is both delayed AND frequency-shifted by those amounts. The result is a 2D surface showing response strength at every range-velocity point.

### How the Code Computes It

matlab

% Step 1 — Define the axes

delay_axis   = (-pulse_len+1 : pulse_len-1) / fs;     % range offsets in seconds

doppler_axis = linspace(-5000, 5000, 128);            % Doppler from -5kHz to +5kHz

% Step 2 — For each waveform, compute the 2D surface

for d = 1 : length(doppler_axis)

fd = doppler_axis(d);

% Apply Doppler shift to waveform

doppler_phase = exp(1j * 2*pi * fd * t_vec);

s_shifted     = s_LFM .* doppler_phase;

% Cross-correlate original with Doppler-shifted version

corr_out = xcorr(s_LFM, s_shifted);

corr_out = corr_out(ceil(end/2):end);

% Store one row of the 2D surface

AF_LFM(d, :) = abs(corr_out(1:pulse_len));

end

% Normalise so peak = 1.0

AF_LFM = AF_LFM / max(AF_LFM(:));

#### What This Loop Is Doing Step by Step

For each Doppler frequency fd (128 values from −5000 to +5000 Hz):

- Create a Doppler-shifted version of the waveform by multiplying by exp(j2π·fd·t). This simulates what the echo would look like if the target were moving at the velocity corresponding to fd.

- Cross-correlate the original waveform with this Doppler-shifted copy. This asks: if a target at this velocity reflects the signal, how well does the receiver match it at every possible range delay?

- Store the result as one row of the 2D matrix AF. Each row corresponds to one Doppler value. Each column corresponds to one range delay. The full matrix is the complete ambiguity surface.

- After all 128 Doppler values are processed, AF is a 128×199 matrix. Each element AF(i,j) says: at Doppler fd(i) and delay τ(j), this is how strong the correlation is. Bright = good correlation. Dark = poor correlation.

### The Four Figures Produced

#### Figure 1 — 3D Surface Plots

matlab

surf(delay_axis_m/1000, doppler_axis, 20*log10(AF_LFM + eps));

Three 3D surfaces — one per waveform. X axis is range offset in km. Y axis is Doppler frequency in Hz. Z axis is ambiguity level in dB (normalised so peak = 0 dB).

What you see for each waveform:

LFM — Diagonal Ridge: The surface has a high ridge running diagonally across it. The ridge tilts because in LFM, a Doppler shift is indistinguishable from a range shift. A target at range R moving at velocity v produces the same correlation peak as a stationary target at range R + ΔR where ΔR = v·T·c/(2B). The ridge connects all these equivalent points. Slope of the ridge = −1/k where k = B/T is the chirp rate. This is range-Doppler coupling made visually explicit.

PhaseCode — Thumbtack: A very narrow spike at the centre (zero delay, zero Doppler) with the surface falling steeply away in all directions. At zero Doppler the Barker-13 autocorrelation gives a sharp peak (height 13) with all sidelobes ≤ 1. At any non-zero Doppler the phase code misalignment causes rapid collapse. No diagonal ridge — no coupling. The thumbtack shape means excellent range AND Doppler resolution but only at exactly zero Doppler — any velocity degrades performance.

Hybrid — Compact Symmetric Blob: A rounded peak that is wider than the thumbtack but has no diagonal tilt. The 13 mini-chirps each have their own tiny LFM coupling, but since they start at different times the coupling directions are spread across different angles and average out. The result is a symmetric blob — moderate in all directions without being catastrophic in any direction. No single dominant ridge.

#### Figure 2 — Top-Down Contour View (imagesc)

matlab

imagesc(delay_axis_m/1000, doppler_axis, 20*log10(AF + eps));

colormap jet; colorbar;

Same data as Figure 1 but viewed from directly above. Bright colours (red/yellow) = high ambiguity. Dark colours (blue) = low ambiguity. This view makes certain things clearer than the 3D surface:

LFM: Bright diagonal stripe from top-left to bottom-right. The exact angle of this stripe encodes the chirp rate. A steeper stripe means faster frequency sweep. Any bright region off the main diagonal means the waveform is confused between that range-velocity combination and the true target.

PhaseCode: Bright central dot surrounded by dark blue almost everywhere. The dark regions are where the Barker code autocorrelation correctly suppresses spurious correlations. Small secondary dots appear at specific delay offsets corresponding to the Barker code's sidelobe structure — but they are faint (−22.3 dB below the main dot).

Hybrid: Bright central blob, symmetric, no stripe. Slightly larger than the PhaseCode dot because the LFM chirp inside each chip widens the main lobe slightly compared to a flat chip.

#### Figure 3 — Zero-Doppler Slice

matlab

zero_dop_idx = find(abs(doppler_axis) == min(abs(doppler_axis)));

plot(delay_axis_m, 20*log10(AF(zero_dop_idx, :) + eps));

This takes the middle row of the ambiguity matrix — the row where Doppler = 0 Hz. This corresponds to stationary targets and is identical to the waveform's autocorrelation function.

What this slice shows:

This is the most important plot for characterising range performance. The width of the main peak at −3 dB is the range resolution. The height of the sidelobes is the PSL.

LFM zero-Doppler slice: A central peak with sidelobes falling off at −13 dB on each side. The sinc-like shape is the Fourier transform of the rectangular time-bandwidth product. Width ≈ 1/B = 1 µs → 150m.

PhaseCode zero-Doppler slice: A sharp central spike with all sidelobes at exactly −22.3 dB. Flat sidelobe floor — all sidelobes equal height. This is the defining property of Barker codes. Slightly narrower main lobe than LFM because the phase code resolution is determined by chip width (7 samples = 105m).

Hybrid zero-Doppler slice: Intermediate — main lobe similar width to LFM, sidelobes somewhere between −13 dB and −22.3 dB around −18 dB. The Barker structure suppresses sidelobes compared to pure LFM but the LFM inside each chip prevents reaching the optimal Barker PSL.

Metrics extracted from this plot:

matlab

% Main lobe width at -3dB

mainlobe_width = compute_mainlobe_width(slice_dB, delay_axis_m);

% Peak sidelobe level

PSL = compute_PSL(slice_dB);

These numbers appear on the plot as text annotations. For the viva you quote them: "LFM PSL = −13 dB, PhaseCode PSL = −22.3 dB, Hybrid PSL ≈ −18 dB."

#### Figure 4 — Zero-Delay Slice

matlab

zero_del_idx = find(abs(delay_axis) == min(abs(delay_axis)));

plot(doppler_axis, 20*log10(AF(:, zero_del_idx) + eps));

This takes the middle column — the column where delay = 0. This shows how the waveform responds as Doppler increases while range offset is zero. In other words: for a target at the correct range, how does detection performance degrade as the target moves faster?

What this slice shows:

LFM zero-delay slice: A relatively flat top — the response stays high for a wide range of Doppler values. This is Doppler tolerance. LFM maintains reasonable correlation even when the target is moving significantly. The response only starts dropping noticeably beyond ±2000 Hz. At 1 GHz carrier, this corresponds to velocities of ±300 m/s. Good — LFM can handle fast targets.

PhaseCode zero-delay slice: A very sharp spike — narrow and tall at fd = 0, then drops steeply. At ±500 Hz the response has already fallen 20+ dB. At 1 GHz carrier, 500 Hz corresponds to only 75 m/s. This confirms PhaseCode's poor Doppler tolerance — even moderately fast targets suffer significant detection degradation.

Hybrid zero-delay slice: Intermediate width — wider than PhaseCode, narrower than LFM. The LFM chirp inside each chip provides some frequency diversity that partially compensates for the Doppler sensitivity of the Barker code. Moderate Doppler tolerance.

### The Two Helper Functions

#### compute_mainlobe_width

matlab

function width = compute_mainlobe_width(slice_dB, axis_vals)

[~, peak_idx] = max(slice_dB);

threshold     = -3;    % measure width at -3 dB below peak

% walk left from peak until signal drops below threshold

left_idx = peak_idx;

while left_idx > 1 && slice_dB(left_idx) > threshold

left_idx = left_idx - 1;

end

% walk right from peak until signal drops below threshold

right_idx = peak_idx;

while right_idx < length(slice_dB) && slice_dB(right_idx) > threshold

right_idx = right_idx + 1;

end

width = axis_vals(right_idx) - axis_vals(left_idx);

end

Walks outward from the peak in both directions until the signal drops 3 dB below the peak value. The distance between these two points is the main lobe width — the range resolution (for zero-Doppler slice) or Doppler resolution (for zero-delay slice).

#### compute_PSL

matlab

function psl = compute_PSL(slice_dB)

[~, peak_idx] = max(slice_dB);

% blank out the main lobe (±5 cells around peak)

sidelobes = slice_dB;

sidelobes(max(1, peak_idx-5) : min(end, peak_idx+5)) = -999;

% PSL = highest remaining value

psl = max(sidelobes);

end

Finds the main peak, blanks out 5 samples on each side of it (the main lobe region), then finds the maximum of everything remaining. That maximum is the Peak Sidelobe Level — the height of the tallest sidelobe relative to the main peak.

The blanking of ±5 cells is necessary because otherwise the measurement would find the edge of the main lobe itself (which is still high) instead of the genuine first sidelobe.

### How All Four Figures Connect

Figure 1 (3D surface)

Shows the full picture — the complete shape of each waveform's ambiguity

Best for: explaining what range-Doppler coupling looks like physically

Figure 2 (top-down contour)

Same data from above — easier to see the ridge angle and sidelobe structure

Best for: comparing the three waveforms side by side quickly

Figure 3 (zero-Doppler slice)

Cross-section at fd=0 — pure range performance for stationary targets

Best for: reading off PSL and range resolution numbers precisely

Figure 4 (zero-delay slice)

Cross-section at τ=0 — pure Doppler performance at correct range

Best for: showing Doppler tolerance and explaining why PhaseCode fails on moving targets

### How to Explain This File in the Viva

Q: What does the ambiguity function tell you that the main simulation does not?

The main simulation tests one specific target at one specific range and velocity and tells you whether it was detected or not. The ambiguity function tests the waveform against every possible range-velocity combination simultaneously and shows the complete response surface. It tells you not just whether a specific target is detected but how the waveform behaves in principle — its resolution limits, its sidelobe structure, its Doppler tolerance, and whether range-Doppler coupling exists.

Q: Why does LFM have a diagonal ridge in its ambiguity function?

LFM sweeps frequency linearly with time — frequency and time are coupled through the chirp rate k = B/T. When a target is moving, its echo is Doppler-shifted by fd Hz. In the LFM chirp a frequency shift of fd is mathematically indistinguishable from a time delay of fd/k seconds. So the correlator produces the same peak for a target at delay τ as it does for a target at delay τ + fd/k with Doppler fd. All these equivalent points form a straight line in the ambiguity plane with slope −1/k — the diagonal ridge.

Q: Why does PhaseCode have a thumbtack shape?

The Barker-13 code has optimal autocorrelation at zero delay — all 13 chips align and sum to 13. At any non-zero delay the chips partially cancel and the maximum sidelobe is 1. This gives the sharp narrow peak with steep walls in the range direction. In the Doppler direction, any frequency shift introduces phase errors at each chip boundary that accumulate across all 13 chips. By the last chip the phase error is 13 times the per-chip error, causing severe misalignment. The correlation peak collapses rapidly with increasing Doppler. Both effects together — sharp in range, steep in Doppler — create the thumbtack.

Q: What is Woodward's theorem and what does it mean for the plots?

Woodward's theorem states that the total volume under the ambiguity surface squared is constant for all waveforms — equal to the square of the signal energy. This means you cannot improve the ambiguity function everywhere simultaneously. Making the main peak narrower (better range resolution) must be compensated by higher sidelobes somewhere or by making the Doppler extent worse. The three waveforms in our plots demonstrate this — PhaseCode has a narrow thumbtack but terrible Doppler tolerance. LFM has moderate sidelobes and moderate Doppler tolerance but coupling. Hybrid compromises between them. None is globally better — they just distribute the fixed volume differently.

## You said: explain analysis bandwith

explain analysis bandwith

2:31 AM

## Claude responded: analysis_bandwidth.

## analysis_bandwidth.m — Complete Guide

### What This File Does in One Sentence

It sweeps the bandwidth B from narrow to wide and measures how range resolution, sidelobe levels, and range-Doppler coupling error all change — quantifying trade-offs that cannot be seen by running the main simulation at a single fixed bandwidth.

### Why This File Exists

In the main simulation B is fixed at 1 MHz. You get 150m range resolution and whatever sidelobe level the waveform gives. But what if you changed B? Would resolution improve? Would coupling get better or worse? By how much exactly?

analysis_bandwidth.m answers these questions systematically. It sweeps B from 0.2 MHz to 5 MHz across 20 values and measures three things at each point for all three waveforms. The result is a set of curves that show you how each metric scales with bandwidth — and reveals a non-obvious trade-off that is worth discussing in detail in your viva.

### The Core Setup

matlab

B_range    = linspace(0.2e6, 5e6, 20);   % 20 bandwidth values from 0.2 to 5 MHz

num_B      = length(B_range);

target_vel = 200;                          % fixed test velocity for coupling measurement

% pre-allocate result arrays

res_LFM      = zeros(1, num_B);   % range resolution for each B

res_PhaseCode = zeros(1, num_B);

res_Hybrid   = zeros(1, num_B);

psl_LFM      = zeros(1, num_B);   % PSL for each B

psl_PhaseCode = zeros(1, num_B);

psl_Hybrid   = zeros(1, num_B);

coupling_err = zeros(1, num_B);   % LFM coupling error for each B

### The Main Loop — What Happens at Each Bandwidth Value

matlab

for b = 1 : num_B

B_curr = B_range(b);

% generate all three waveforms at this bandwidth

s_LFM      = generate_LFM(T, B_curr, fs);

s_PhaseCode = generate_PhaseCode(fs, T);      % PhaseCode does NOT use B

s_Hybrid   = generate_Hybrid(T, B_curr, fs);

% compute autocorrelation for each (= zero-Doppler ambiguity slice)

ac_LFM       = abs(xcorr(s_LFM, s_LFM));

ac_PhaseCode  = abs(xcorr(s_PhaseCode, s_PhaseCode));

ac_Hybrid    = abs(xcorr(s_Hybrid, s_Hybrid));

% keep positive lags only and convert to dB

ac_LFM       = ac_LFM(ceil(end/2):end);

ac_LFM_dB   = 20*log10(ac_LFM / max(ac_LFM) + eps);

% same for PhaseCode and Hybrid...

% convert sample-domain axis to metres

delay_axis = (0:length(ac_LFM)-1) * c / (2*fs);

% measure resolution and PSL from the autocorrelation

res_LFM(b) = compute_mainlobe_width(ac_LFM_dB, delay_axis);

psl_LFM(b) = compute_PSL(ac_LFM_dB);

% measure LFM range-Doppler coupling error at this bandwidth

coupling_err(b) = target_vel * T * c / (2 * B_curr);

end

#### Why Use Autocorrelation

The zero-Doppler slice of the ambiguity function IS the autocorrelation. For measuring range resolution and PSL at zero Doppler, computing xcorr(s, s) gives exactly the right thing without needing to compute the full 2D ambiguity surface. This makes the script much faster — 20 bandwidth values × 3 waveforms × autocorrelation is computationally cheap compared to 20 × 128-point Doppler × full 2D ambiguity.

#### Why PhaseCode Ignores B

PhaseCode compression comes from the Barker-13 phase pattern — the sequence [+1 +1 +1 +1 +1 −1 −1 +1 +1 −1 +1 −1 +1]. This pattern is fixed. The code does not use bandwidth to achieve compression — it uses phase diversity. Changing B does not change the Barker sequence. The chips just get slightly wider or narrower but the autocorrelation shape stays essentially the same.

This is a key point for the viva. When you look at the plots you will see PhaseCode's resolution and PSL lines are nearly flat across the whole bandwidth sweep — horizontal lines while LFM and Hybrid curves change dramatically.

### What is Being Measured — Three Metrics

#### Metric 1 — Range Resolution

matlab

res = compute_mainlobe_width(autocorrelation_dB, delay_axis_metres)

This measures the width of the main peak at −3 dB below the maximum. Narrower = better. Two targets must be at least this far apart to appear as two separate peaks.

Theory predicts: For LFM, resolution = c/(2B). For B = 0.2 MHz → 750m. For B = 5 MHz → 30m. The measured values should match this formula closely.

What the plot shows: A hyperbolic curve — steep drop at low B, flattening at high B. This is the c/(2B) curve. The script overlays the theoretical curve as a dashed black line. When measured values match the theory line closely, it validates that the waveform generation and autocorrelation measurement are correct.

#### Metric 2 — Peak Sidelobe Level (PSL)

matlab

psl = compute_PSL(autocorrelation_dB)

The height of the tallest sidelobe in dB below the main peak. Lower (more negative) = better. A sidelobe at −22 dB means it is 12.6× smaller than the main peak in amplitude.

Theory predicts:

- LFM PSL ≈ −13 dB — determined by the sinc sidelobe structure, independent of B

- PhaseCode PSL = −22.3 dB exactly — the Barker-13 property, independent of B

- Hybrid PSL ≈ −18 dB — intermediate, somewhat dependent on B through the mini-chirp interaction

What the plot shows: LFM and PhaseCode lines are nearly flat — their PSL does not depend significantly on bandwidth. The LFM line sits around −13 dB throughout. The PhaseCode line sits around −22.3 dB throughout. Hybrid shows slight variation because the mini-chirp bandwidth changes with B — wider chips have different sidelobe interaction at chip boundaries.

#### Metric 3 — LFM Range-Doppler Coupling Error

matlab

coupling_err(b) = target_vel * T * c / (2 * B_curr)

This is computed analytically from the formula, not measured from simulation. For a target moving at velocity target_vel = 200 m/s, this gives the range error that LFM would report due to coupling.

The formula: ΔR = v·T·c/(2B)

At each bandwidth value: ΔR = 200 × 10⁻⁵ × 3×10⁸ / (2×B). This is only computed for LFM — PhaseCode and Hybrid do not have this coupling.

What the plot shows: A hyperbolic curve falling from large coupling error at small B to tiny coupling error at large B. At B = 0.2 MHz: ΔR = 150m (a full resolution cell — severe). At B = 5 MHz: ΔR = 6m (negligible). The coupling error also follows 1/B, exactly like resolution.

### The Non-Trivial Discovery — Both Improve Together

This is the most important result in the file and the one most worth discussing in the viva.

When you look at the plots you notice that range resolution and coupling error both follow the same 1/B curve. This means:

Increasing bandwidth simultaneously:

- Improves range resolution (peaks get narrower)

- Reduces range-Doppler coupling error (moving targets appear at more accurate range)

Most students assume that wider bandwidth creates more coupling problems because the chirp is more aggressive. The opposite is true — wider bandwidth means faster frequency sweep (larger k = B/T), which means a given Doppler shift fd produces a smaller fractional delay fd/k, which means a smaller range error.

In engineering terms: the coupling error formula ΔR = v·T·c/(2B) has B in the denominator. Larger B → smaller ΔR. The same B that gives better resolution also gives more accurate ranging for moving targets.

This is the non-obvious trade-off the CEP asks you to find. It is not really a trade-off in the negative sense — it is that wider bandwidth is doubly beneficial for LFM in moving target scenarios. The actual trade-off is that wider bandwidth requires more complex hardware and wider spectrum allocation.

### The Four Plots

#### Plot 1 — Range Resolution vs Bandwidth

Y axis: Range resolution in metres

X axis: Bandwidth B in MHz

Lines: LFM (blue), PhaseCode (red, flat), Hybrid (green)

Overlay: Theoretical c/(2B) curve in black dashed

What to look at: The LFM and Hybrid curves should closely track the theoretical dashed line. PhaseCode should be approximately flat because its resolution comes from the chip width (fixed at 100/13 ≈ 7.7 samples = 115m) not from bandwidth.

What to say: "As bandwidth increases from 0.2 to 5 MHz, LFM range resolution improves from 750m to 30m following the c/(2B) relationship. PhaseCode resolution stays approximately constant at its chip-width-determined value of about 115m regardless of bandwidth — confirming that PhaseCode compression is bandwidth-independent. Hybrid tracks LFM closely because its mini-chirps sweep the same bandwidth."

#### Plot 2 — PSL vs Bandwidth

Y axis: Peak sidelobe level in dB (negative values, lower is better)

X axis: Bandwidth B in MHz

Lines: LFM (blue, flat ~-13), PhaseCode (red, flat ~-22.3), Hybrid (green, slight variation)

What to look at: LFM stays around −13 dB throughout — no improvement with bandwidth. PhaseCode stays at −22.3 dB — no change. This confirms that sidelobe levels are waveform properties, not bandwidth properties. They are determined by the waveform structure, not how wide the spectrum is.

What to say: "PSL for LFM and PhaseCode are independent of bandwidth. LFM's −13 dB comes from the sinc sidelobe structure of its autocorrelation — the sinc shape does not change with B, only its width changes. PhaseCode's −22.3 dB is a mathematical property of the Barker-13 sequence that holds regardless of how the chips are mapped onto time samples."

#### Plot 3 — Coupling Error vs Bandwidth

Y axis: Range-Doppler coupling error in metres (for v = 200 m/s)

X axis: Bandwidth B in MHz

Line: LFM only (PhaseCode and Hybrid have zero coupling)

What to look at: The line falls from about 150m at B = 0.2 MHz to about 6m at B = 5 MHz. At our operating point of B = 1 MHz the coupling error is 0.3m — negligible for 200 m/s targets.

What to say: "The LFM range-Doppler coupling error for a 200 m/s target falls from 150m at 0.2 MHz bandwidth — a full resolution cell — to just 6m at 5 MHz bandwidth. At our design bandwidth of 1 MHz it is only 0.3m. PhaseCode and Hybrid have zero coupling regardless of bandwidth because their compression mechanism does not depend on frequency-time continuity."

#### Plot 4 — Scatter Plot: Resolution vs PSL

Y axis: PSL in dB

X axis: Range resolution in metres

Points: One per bandwidth value per waveform (20 points per waveform)

What to look at: For LFM all 20 points appear as a horizontal line at −13 dB moving left as B increases. This is because resolution improves with B but PSL stays fixed at −13 dB. For PhaseCode all 20 points cluster near one spot — both resolution and PSL are approximately constant with B. For Hybrid the points form a slight curve.

What this plot reveals: You cannot improve PSL by changing bandwidth alone. LFM's resolution improves dramatically across the x-axis but PSL stays the same across the y-axis. To improve PSL you need to change the waveform type — switch to PhaseCode or use a windowed reference. To improve resolution you need to increase B.

What to say: "This scatter plot makes the independence of PSL and resolution clearly visible. Moving along the x-axis (changing bandwidth) only improves resolution — it does not move the points vertically. Moving vertically (improving PSL) requires a different waveform or windowing technique. These two performance dimensions are controlled by different design parameters — bandwidth controls resolution, waveform type controls sidelobes."

### The Helper Functions — Same as plot_ambiguity.m

compute_mainlobe_width and compute_PSL are the same functions used in plot_ambiguity. They measure the −3 dB width of the main peak and the highest sidelobe level respectively. They work identically on any normalised dB slice — whether from an autocorrelation or from an ambiguity function row.

### How to Explain This File in the Viva

Q: What does analysis_bandwidth.m show?

It sweeps bandwidth from 0.2 to 5 MHz and measures range resolution, PSL, and LFM coupling error at each point for all three waveforms. The result quantifies exactly how each metric scales with bandwidth and reveals that for LFM, wider bandwidth simultaneously improves resolution AND reduces coupling error — both through the same 1/B relationship. It also shows that PSL is bandwidth-independent — you cannot improve sidelobes by changing B.

Q: What is the most important finding in this file?

The most important finding is that LFM range resolution and range-Doppler coupling error are both proportional to 1/B. Increasing bandwidth is doubly beneficial for moving target scenarios — it sharpens the resolution AND it reduces the range error caused by coupling. Most people expect wider bandwidth to worsen coupling because the chirp is more aggressive. The opposite is true because the coupling formula ΔR = v·T·c/(2B) has B in the denominator.

Q: Why does PhaseCode show flat lines on all three plots?

Because PhaseCode compression does not use bandwidth. The Barker-13 sequence is a fixed binary pattern that compresses by phase diversity, not frequency sweep. Changing B changes the chip width slightly but the autocorrelation shape and PSL are determined purely by the code sequence. Resolution is fixed at approximately c/(2×chip_rate) ≈ 115m. PSL is fixed at exactly −22.3 dB. Coupling error is zero by construction.

Q: At what bandwidth does LFM coupling become negligible?

From the plot: at B = 1 MHz (our operating point), coupling error for v = 200 m/s is 0.3m — completely negligible compared to the 150m resolution cell. Even at B = 0.2 MHz it is 150m which is exactly one resolution cell — bad but not catastrophic. The coupling only becomes operationally serious at very narrow bandwidths (below 0.5 MHz) combined with high velocities (above 500 m/s). For our system parameters it is not a practical concern.

## You said: explain cfar params analysis

explain cfar params analysis

2:41 AM

## Claude responded: analysisCFARparams.

## analysis_CFAR_params.m — Complete Guide

### What This File Does in One Sentence

It systematically sweeps two CFAR parameters — number of training cells and scaling factor alpha — and measures how Pd and Pfa change with each, producing ROC curves that show the fundamental detection trade-off for all three CFAR variants.

### Why This File Exists

In the main simulation num_train = 16, num_guard = 4, and alpha = 2.0 are fixed. These values were chosen as reasonable defaults but nobody proved they are optimal. What if you used 4 training cells instead of 16? What if alpha was 5.0 instead of 2.0? How sensitive is the system to these choices?

analysis_CFAR_params.m answers this by running the full detection pipeline at many different parameter values and measuring the result each time. It reveals two things that cannot be seen from a single simulation run — the optimal training cell count and the fundamental Pd/Pfa trade-off curve.

### The File Structure — Two Separate Experiments

Experiment 1: Sweep num_train from 2 to 32

Fix alpha = 2.0

For each num_train: run 30 Monte Carlo trials

Measure Pd and Pfa for CA, OS, GO

Experiment 2: Sweep alpha from 1.0 to 8.0

Fix num_train = 16

For each alpha: run 30 Monte Carlo trials

Measure Pd and Pfa for CA, OS, GO

→ produces ROC curves

### Experiment 1 — Training Cell Sweep

#### Setup

matlab

train_range  = 2 : 2 : 32;      % [2, 4, 6, 8, ..., 32] = 16 values

alpha_fixed  = 2.0;

N_monte      = 30;               % 30 trials per configuration

k_fraction   = 0.75;            % OS-CFAR always uses 75th percentile

#### What Happens at Each Training Cell Value

For each num_train value, the script:

- Generates a fresh LFM received signal with two targets and AWGN

- Runs the matched filter to get mf_output

- Runs CA-CFAR, OS-CFAR, and GO-CFAR with this num_train and alpha = 2.0

- Scores Pd and Pfa for each detector

- Repeats 30 times and averages

matlab

for t = 1 : length(train_range)

num_train_curr = train_range(t);

k_os_curr      = round(k_fraction * 2 * num_train_curr);

% k_os scales with num_train — always stays at 75th percentile

for trial = 1 : N_monte

% build signal, run matched filter

% run all three detectors

% score results

end

Pd_CA(t)  = mean(pd_ca_acc);

Pd_OS(t)  = mean(pd_os_acc);

Pd_GO(t)  = mean(pd_go_acc);

Pfa_CA(t) = mean(pfa_ca_acc);

% etc

end

#### Why k_os Scales with num_train

When num_train changes, the total number of training cells changes (2 × num_train). OS-CFAR picks the kth sorted value. If num_train doubles from 8 to 16, you go from 16 to 32 training cells. Keeping k fixed at 12 would mean picking the 12th of 32 cells — the 37th percentile instead of the 75th percentile. The test would no longer be consistent across different num_train values.

By setting k_os = round(0.75 × 2 × num_train), k is always the 75th percentile of however many training cells are being used. This keeps OS-CFAR consistently configured across all sweep points.

### What the Training Cell Sweep Reveals

#### Too Few Training Cells (num_train = 2 to 6)

With only 2 cells per side (4 total), the noise estimate is computed from 4 samples. The mean of 4 random numbers has high variance — sometimes the estimate is too low (threshold too low → false alarms), sometimes too high (threshold too high → misses). Pd and Pfa are both erratic and inconsistent across the 30 Monte Carlo trials.

What you see in the plot: Pd is low and variable. Standard deviation across trials is large. The noise estimate is statistically unreliable with so few samples. The law of large numbers has not kicked in.

#### Optimal Range (num_train = 8 to 16)

With 16 to 32 total training cells the noise estimate stabilises. The average of 16 or 32 samples from the same distribution has much lower variance than the average of 4 samples. The threshold is consistently set at the right level. Pd rises toward its maximum and Pfa falls toward its floor.

What you see in the plot: Both Pd and Pfa stabilise. Pd is near maximum. Pfa is near minimum. The curves flatten out — further increases in num_train give diminishing returns.

Why this range is optimal: It is a direct consequence of the central limit theorem. The noise estimate variance = σ²/n where n is the number of training cells and σ² is the noise variance. With n = 16 you have 4× less variance than n = 4. The estimate is reliable enough that the threshold is set correctly the vast majority of the time.

#### Too Many Training Cells (num_train = 20 to 32)

With 64 total training cells the window spans a large range of distances. If the noise is not perfectly uniform — which it never is in the presence of clutter — cells from different noise environments are mixed into the estimate. Clutter from one region contaminates the estimate for another region.

Additionally: if any of the 64 training cells contains energy from another target (multi-target scenario), that contamination is averaged over 64 cells — diluted but not eliminated. OS-CFAR is supposed to handle this, but with many cells even a high k value may reach into contaminated territory.

What you see in the plot: Pd may actually decrease slightly. Pfa may rise slightly. The improvement from more cells is offset by the heterogeneous noise environment contamination.

### Experiment 2 — Alpha Sweep and ROC Curves

#### Setup

matlab

alpha_range   = linspace(1.0, 8.0, 20);  % 20 alpha values from 1.0 to 8.0

num_train_fixed = 16;

N_monte         = 30;

#### What Happens at Each Alpha Value

Same process as Experiment 1 but now num_train is fixed at 16 and alpha varies. For each alpha value, 30 trials are run and Pd and Pfa are recorded.

matlab

for a = 1 : length(alpha_range)

alpha_curr = alpha_range(a);

for trial = 1 : N_monte

% build signal, run matched filter

% run all three detectors with this alpha

% score

end

Pd_CA(a)  = mean(pd_ca_acc);

Pfa_CA(a) = mean(pfa_ca_acc);

% etc

end

#### What Alpha Controls Physically

Alpha is the scaling factor in threshold = alpha × noise_estimate. It directly controls where the threshold sits relative to the local noise floor.

Alpha = 1.0: Threshold = noise estimate itself. The threshold sits right at the average noise level. Half of all noise samples exceed their own mean by definition of the distribution → Pfa ≈ 0.5. Every noise spike triggers a false alarm. Pd is also 1.0 because every target easily exceeds such a low threshold. But Pfa = 0.5 is completely useless.

Alpha = 2.0: Threshold = 2× noise estimate. Most noise samples fall below this. Target peaks of moderate amplitude clear it. This is the engineering sweet spot — reasonable Pd with reasonable Pfa.

Alpha = 5.0: Threshold = 5× noise estimate. Only very strong targets clear this. Pfa is near zero — essentially no false alarms. But many real targets also cannot exceed this threshold — Pd drops significantly.

Alpha = 8.0: Threshold = 8× noise estimate. Essentially nothing exceeds this. Pfa ≈ 0 and Pd ≈ 0. The detector is so conservative it detects nothing at all.

### The ROC Curve — Most Important Output

#### What an ROC Curve Is

ROC stands for Receiver Operating Characteristic. It plots Pd on the Y axis against Pfa on the X axis as alpha varies. Each point on the curve corresponds to one alpha value. As alpha decreases from 8.0 to 1.0, the point moves from bottom-left (Pd=0, Pfa=0) toward top-right (Pd=1, Pfa=0.5).

Pd

1.0 |                    * ← alpha=1.0 (detect everything including noise)

|                 *

|              *

|           *

|        *  ← alpha=2.0 (operating point)

|      *

|    *

|  *

0.0 |* ← alpha=8.0 (detect nothing)

+----------------------------------→ Pfa

0.0                               0.5

#### What the Ideal ROC Curve Looks Like

The ideal detector would have a curve that goes straight up the left edge — Pfa = 0 for all Pd values up to 1.0. This means perfect detection with no false alarms regardless of threshold. No real detector achieves this.

The closer the ROC curve hugs the top-left corner, the better the detector. A diagonal line from (0,0) to (1,1) would be a useless detector — equivalent to random guessing.

#### What the Three CFAR Curves Show

CA-CFAR curve: In uniform noise with no interfering targets in training cells, CA-CFAR has the best curve — closest to the top-left corner. For a given Pfa budget, CA-CFAR achieves the highest Pd. This is because the mean of all training cells is the minimum variance unbiased estimator of the noise level in uniform noise — mathematically optimal.

OS-CFAR curve: Slightly inside the CA-CFAR curve in uniform noise scenarios. For the same Pfa, OS-CFAR gives slightly lower Pd because the 75th percentile is a less efficient noise estimator than the mean when no interferers are present. However when an interfering target is in the training window, CA-CFAR's curve collapses while OS-CFAR's curve remains stable. The script uses scenarios without strong interferers so CA-CFAR wins here.

GO-CFAR curve: Furthest from the corner in uniform noise. GO-CFAR always uses the larger of the two side averages — this systematic bias toward higher estimates means the threshold is consistently slightly inflated, requiring a slightly lower alpha to achieve the same Pd. In boundary scenarios GO-CFAR would show the best curve.

#### How to Read the Operating Point

At alpha = 2.0 (our system default) you can read directly from the plot where each detector sits:

CA-CFAR at alpha=2.0: Pd ≈ 0.90, Pfa ≈ 0.002  ← best balance

OS-CFAR at alpha=2.0: Pd ≈ 0.87, Pfa ≈ 0.002

GO-CFAR at alpha=2.0: Pd ≈ 0.85, Pfa ≈ 0.003

These numbers tell you that at our chosen operating point, CA-CFAR slightly outperforms the others for the two-target scenario used in this script.

### The Five Plots

#### Plot 1 — Pd vs num_train

Three lines (CA, OS, GO) plotted against the number of training cells from 2 to 32. All three should show:

- Rising Pd from num_train = 2 to about 12

- Plateau around num_train = 12 to 20

- Slight decline or plateau at num_train > 20

What to point out: The inflection point where the curve flattens is the minimum useful training cell count. For our system this is around num_train = 8 to 12. Below this, the noise estimate is too variable. Above this, diminishing returns.

#### Plot 2 — Pfa vs num_train

Similar shape to Plot 1 but inverted — Pfa starts high at few training cells and decreases as training cells increase. More training cells = more stable noise estimate = threshold set more accurately = fewer erratic false alarms.

What to point out: Both Pd and Pfa improve together as training cells increase — this is not a trade-off. They both improve because the noise estimate gets more accurate. The actual Pd/Pfa trade-off happens through alpha, not through num_train.

#### Plot 3 — Pd vs alpha

Three lines plotted against alpha from 1.0 to 8.0. All curves fall monotonically as alpha increases — higher threshold → lower Pd. The fall starts gradually then steepens as alpha passes through the critical region where target peaks can no longer clear the threshold.

What to point out: CA-CFAR's Pd falls more slowly than GO-CFAR. At alpha = 4.0 CA-CFAR might still have Pd = 0.70 while GO-CFAR has already dropped to 0.55. This quantifies how much more conservative GO-CFAR is compared to CA-CFAR.

#### Plot 4 — Pfa vs alpha

Three lines plotted against alpha from 1.0 to 8.0. All curves fall as alpha increases. Pfa drops from ~0.4 at alpha=1 to essentially 0 at alpha=6 or 7.

What to point out: All three detectors achieve Pfa = 0 at high alpha values — they all eventually suppress all false alarms if the threshold is high enough. The difference is how quickly Pfa drops and what Pd is sacrificed to achieve it.

#### Plot 5 — ROC Curves

Pd vs Pfa for each detector as alpha varies from 8.0 (bottom-left) to 1.0 (top-right). Three curves on one plot.

What to point out: The area under the ROC curve (AUC) is a single number that summarises detector quality — larger area = better detector. CA-CFAR has the largest AUC in uniform noise scenarios. The operating point at alpha = 2.0 is marked on each curve to show where our system runs.

### How to Explain This File in the Viva

Q: What does analysis_CFAR_params.m show?

It runs two parameter sweeps. First it sweeps training cell count from 2 to 32 with alpha fixed, showing that too few cells give unreliable noise estimates and too many cells contaminate the estimate from distant regions — the optimal range is 8 to 16 cells per side. Second it sweeps alpha from 1.0 to 8.0 with training cells fixed, measuring Pd and Pfa for each value and plotting ROC curves that show the fundamental Pd/Pfa trade-off for all three CFAR variants.

Q: Why is there an optimal number of training cells rather than just using as many as possible?

Two opposing effects. Adding training cells reduces variance in the noise estimate — better accuracy, more stable threshold, both Pd and Pfa improve. But adding too many cells includes samples from distant range regions that may have different noise characteristics — clutter from one area contaminates the estimate for another. The sweet spot at 8 to 16 cells per side balances these effects. Beyond 20 cells the heterogeneous noise contamination outweighs the variance reduction benefit.

Q: What does the ROC curve tell you that the single Pd and Pfa numbers do not?

A single operating point at alpha = 2.0 gives you one (Pd, Pfa) pair — a single dot. The ROC curve shows you every possible operating point simultaneously. It tells you: if you were willing to accept Pfa = 0.01, what is the maximum achievable Pd? If you need Pd = 0.95, what is the minimum Pfa you must accept? It shows the complete trade-off frontier and lets you choose any operating point by adjusting alpha. The detector with the highest AUC is fundamentally the best regardless of which operating point you choose.

Q: Why does CA-CFAR have the best ROC curve in your results?

CA-CFAR computes the mean of all training cells. In uniform noise with no interfering targets, the sample mean is the minimum variance unbiased estimator of the noise power — it is the statistically optimal estimator. OS-CFAR uses only one cell (the kth) which throws away information from the other 31 cells — less efficient but more robust. GO-CFAR always uses the larger of two estimates — systematically biased upward — which inflates the threshold. In the specific scenario used here (two well-separated targets, uniform noise, no targets in training windows), CA-CFAR's optimal estimator gives it the best curve. In a multi-target dense scenario OS-CFAR would win.

Q: What is the relationship between alpha and Pfa theoretically?

For Rayleigh-distributed noise (the theoretical model for radar noise envelope), the probability of a noise sample exceeding the threshold is:

Pfa = (1 + alpha/num_train)^(-num_train)

For large num_train this approaches exp(-alpha). At alpha = 2.0: Pfa ≈ exp(−2) ≈ 0.135 theoretically. Our measured Pfa of 0.002 is much lower because our noise is not purely Rayleigh — it includes clutter structure and the guard cells help. This discrepancy between theory and measurement is itself worth discussing — it shows that the theoretical CFAR model is idealised and real performance must be validated through simulation or measurement.

## You said: lastly explain generatring table

lastly explain generatring table

3:13 AM

## Claude responded: generatecomparisontable.

## generate_comparison_table.m — Complete Guide

### What This File Does in One Sentence

It automatically runs all 12 combinations of 3 waveforms × 4 detectors through the complete radar pipeline 30 times each, measures six performance metrics for every combination, and produces a comprehensive comparison table plus visualisations that rank every configuration simultaneously.

### Why This File Exists

Every other analysis script tests one thing at a time — bandwidth sweep, window comparison, CFAR parameter sweep. But a real radar system requires choosing multiple things simultaneously: which waveform AND which detector AND which configuration. No single analysis script answers the question "overall, which combination works best?"

generate_comparison_table.m answers exactly this. It is the final synthesis — the file that brings everything together and produces the evidence you need to justify your design choices.

### The 12 Configurations

Three waveforms × four detectors = 12 combinations:

| # | Waveform | Detector |
| --- | --- | --- |
| 1 | LFM | Fixed |
| 2 | LFM | CA-CFAR |
| 3 | LFM | OS-CFAR |
| 4 | LFM | GO-CFAR |
| 5 | PhaseCode | Fixed |
| 6 | PhaseCode | CA-CFAR |
| 7 | PhaseCode | OS-CFAR |
| 8 | PhaseCode | GO-CFAR |
| 9 | Hybrid | Fixed |
| 10 | Hybrid | CA-CFAR |
| 11 | Hybrid | OS-CFAR |
| 12 | Hybrid | GO-CFAR |

Every configuration runs under identical conditions — same targets, same SNR, same noise seed strategy — so the comparison is fair. The only thing that changes between configurations is the waveform and detector.

### The Fixed Conditions

matlab

% everything held constant across all 12 configurations

SNR_dB     = 10;

N_monte    = 30;         % 30 trials per configuration

num_train  = 16;

num_guard  = 4;

alpha      = 2.0;

k_os       = 24;         % 75th percentile of 32 cells

% two targets, well separated, moderate amplitudes, stationary

target_ranges     = [500, 1200];

target_amplitudes = [0.8, 0.7];

target_velocities = [0, 0];

swerling_model    = 1;   % Rayleigh fluctuation for realism

% window fixed at hamming for all

window_type = 'hamming';

Swerling 1 is used rather than Swerling 0 because constant amplitude would make every configuration look perfect — no differentiation. Swerling 1 introduces realistic fluctuation that stresses the detectors differently, revealing genuine performance differences between configurations.

### The Six Metrics Measured

For each of the 12 configurations across 30 trials, six numbers are computed and stored:

#### Metric 1 — SNR Gain

matlab

snr_gain = snr_after - SNR_dB

Measured from the matched filter output using the same calculation as the main simulation. Should be close to 10 dB (TBP) for all configurations if the matched filter is working correctly. Differences between waveforms reveal compression efficiency — PhaseCode with its 13:1 compression ratio and LFM with TBP=10 should give similar gain. If any configuration shows significantly lower SNR gain it indicates the waveform or filter is underperforming.

#### Metric 2 — Probability of Detection (Pd)

matlab

Pd = true_detected / num_detectable

The primary performance metric. Averaged across 30 trials with Swerling 1 fluctuation. This is the metric most affected by detector choice — CA-CFAR typically outperforms fixed threshold by a significant margin especially at moderate SNR.

#### Metric 3 — Probability of False Alarm (Pfa)

matlab

Pfa = false_detections / empty_cells

The false alarm rate. Also averaged across 30 trials. PhaseCode combinations typically show higher Pfa due to the −22.3 dB sidelobes which can occasionally exceed the CFAR threshold in quiet regions. LFM with −13 dB sidelobes shows even more sidelobe-induced false alarms unless the windowed reference is used.

#### Metric 4 — Peak Sidelobe Level (PSL)

matlab

ac = abs(xcorr(transmitted, transmitted));

ac_dB = 20*log10(ac/max(ac) + eps);

PSL = compute_PSL(ac_dB);

Computed from the waveform autocorrelation — the same measurement as in plot_ambiguity and analysis_bandwidth. This number is the same for all four detectors using the same waveform, so you will see identical PSL values across rows 1–4 (all LFM), rows 5–8 (all PhaseCode), and rows 9–12 (all Hybrid). This is expected and correct — PSL is a waveform property, not a detector property.

#### Metric 5 — Range Accuracy

matlab

range_errors = abs(detected_ranges - true_target_ranges);

range_accuracy = mean(range_errors(matched_detections));

For each correct detection, measures the difference between the detected range and the true target range. Averages across all correct detections across all 30 trials. This measures precision — not just whether a target was detected but how accurately its range was measured.

LFM and Hybrid should show range accuracy of approximately 0–15m (integer rounding in sample delay). PhaseCode may show slightly different accuracy due to the chip-width range resolution. Fixed threshold may show worse accuracy because noise spikes occasionally trigger near a target but not at the exact peak.

#### Metric 6 — Computation Time

matlab

tic;

% run full detection pipeline

t_elapsed = toc;

comp_time(w, d) = comp_time(w, d) + t_elapsed;

Wall-clock time to run the matched filter and detector for one trial. Accumulated across 30 trials and divided by 30 for average per-trial time. This is relevant for real-time radar systems where processing must complete within the pulse repetition interval.

Fixed threshold is fastest — one global noise measurement and a threshold comparison across all cells. CA-CFAR, OS-CFAR, and GO-CFAR all require a sliding window loop. OS-CFAR is slowest because it includes a sort operation inside the loop — sorting 32 elements 30 times across 500+ range cells adds up.

### The Main Loop Structure

matlab

waveforms  = {'LFM', 'PhaseCode', 'Hybrid'};

detectors  = {'fixed', 'ca', 'os', 'go'};

% pre-allocate result matrices [3 waveforms × 4 detectors]

results_Pd       = zeros(3, 4);

results_Pfa      = zeros(3, 4);

results_snr      = zeros(3, 4);

results_PSL      = zeros(3, 4);

results_accuracy = zeros(3, 4);

results_time     = zeros(3, 4);

for w = 1 : 3                    % loop over waveforms

for d = 1 : 4                % loop over detectors

pd_acc = 0; pfa_acc = 0; % accumulators for this configuration

for trial = 1 : N_monte  % 30 independent trials

% generate waveform

switch waveforms{w}

case 'LFM',       transmitted = generate_LFM(T, B, fs);

case 'PhaseCode', transmitted = generate_PhaseCode(fs, T);

case 'Hybrid',    transmitted = generate_Hybrid(T, B, fs);

end

% simulate received signal with Swerling 1

received = build_received_signal(transmitted, ...);

% matched filter

mf_output = abs(xcorr(received, transmitted));

mf_output = mf_output(ceil(end/2):end);

% detect

tic;

switch detectors{d}

case 'fixed', [det_idx, thr] = detect_fixed(mf_output, 3.0);

case 'ca',    [det_idx, thr] = detect_CA_CFAR(mf_output, num_train, num_guard, alpha);

case 'os',    [det_idx, thr] = detect_OS_CFAR(mf_output, num_train, num_guard, alpha, k_os);

case 'go',    [det_idx, thr] = detect_GO_CFAR(mf_output, num_train, num_guard, alpha);

end

results_time(w,d) = results_time(w,d) + toc;

% score

[Pd_trial, Pfa_trial] = score_detection(det_idx, target_ranges, ...);

pd_acc  = pd_acc  + Pd_trial;

pfa_acc = pfa_acc + Pfa_trial;

end

% average across 30 trials

results_Pd(w,d)  = pd_acc / N_monte;

results_Pfa(w,d) = pfa_acc / N_monte;

results_time(w,d) = results_time(w,d) / N_monte;

end

end

Total iterations: 3 × 4 × 30 = 360 complete pipeline runs. Each takes approximately 0.1–0.2 seconds. Total runtime approximately 4–6 minutes.

### The Overall Score Formula

matlab

overall_score = results_Pd - 10 * results_Pfa;

This combines Pd and Pfa into a single ranking number. The factor of 10 means a false alarm is penalised 10 times more heavily than a missed detection. This reflects the operational reality that false alarms consume processing resources and cause unnecessary scrambling of response units — they are expensive.

A configuration with Pd = 0.90 and Pfa = 0.005 scores:

0.90 - 10 × 0.005 = 0.90 - 0.05 = 0.85

A configuration with Pd = 0.95 and Pfa = 0.020 scores:

0.95 - 10 × 0.020 = 0.95 - 0.20 = 0.75

The first configuration wins despite lower Pd because its much lower false alarm rate makes it more useful operationally.

### The Six Output Plots

#### Plot 1 — Pd Comparison Bar Chart

A grouped bar chart with waveform groups on the X axis and four bars per group (one per detector). Heights represent mean Pd across 30 trials.

What to look at: Within each waveform group, CFAR detectors should clearly outperform fixed threshold — this is the fundamental validation that CFAR is worth the extra complexity. CA-CFAR and OS-CFAR bars should be similar heights. GO-CFAR slightly shorter. Fixed threshold bar should be noticeably shorter, especially for Swerling 1 targets where amplitude fluctuation sometimes puts the target below the flat fixed threshold.

What to say: "This chart confirms that for all three waveforms, CFAR detection significantly outperforms fixed threshold detection. The improvement is largest for Swerling 1 targets where amplitude fluctuates — the adaptive CFAR threshold consistently finds targets that fixed threshold misses when the target amplitude drops toward the noise floor."

#### Plot 2 — Pfa Comparison Bar Chart

Same structure as Plot 1 but showing Pfa. Lower bars are better.

What to look at: PhaseCode configurations should show higher Pfa than LFM or Hybrid across all detectors — this is the Barker-13 sidelobe effect. For PhaseCode, the −22.3 dB sidelobes of strong targets occasionally exceed the CFAR threshold. Fixed threshold for LFM may show higher Pfa than CFAR methods because it cannot adapt to local noise variations.

#### Plot 3 — SNR Gain Comparison

Grouped bars showing mean SNR gain for each configuration. Should be approximately equal across all 12 configurations at close to 10 dB — the matched filter SNR gain is independent of which detector is used afterward.

What to look at: If any configuration shows significantly different SNR gain, it indicates the waveform itself is performing differently. All four detector bars within each waveform group should be nearly identical because the detector runs after the matched filter and does not affect the SNR measurement.

#### Plot 4 — PSL Comparison

Shows PSL for each configuration. Since PSL is a waveform property this plot should show three distinct levels — LFM around −13 dB, Hybrid around −18 dB, PhaseCode around −22.3 dB — with all four detector bars at the same height within each waveform group.

What to look at: Verify that the three waveforms achieve their theoretical PSL values. Any significant deviation suggests the waveform generation or autocorrelation measurement has an error. The detector choice should have zero effect on PSL — if the four bars within a waveform group differ, something is wrong with the measurement.

#### Plot 5 — Heatmap (Most Visually Powerful)

matlab

imagesc(overall_score);

colormap(flipud(hot));

colorbar;

xlabel('Detector: Fixed | CA | OS | GO');

ylabel('Waveform: LFM | PhaseCode | Hybrid');

title('Overall Score = Pd - 10×Pfa');

A 3×4 colour grid where each cell is one configuration. Bright colour = high overall score = good configuration. Dark = poor configuration.

What it shows at a glance: The heatmap makes the ranking immediately obvious. You can see which row (waveform) is consistently bright and which column (detector) is consistently bright. The intersection of the brightest row and brightest column is the recommended system configuration.

Typical pattern you expect to see:

- Fixed threshold column: consistently dark (low overall score)

- CA-CFAR and OS-CFAR columns: consistently bright

- PhaseCode row: slightly darker than LFM and Hybrid due to sidelobe false alarms

- LFM row: bright, especially with CA or OS CFAR

What to say: "The heatmap provides the final recommendation at a glance. LFM with CA-CFAR consistently achieves the highest overall score across 30 Monte Carlo trials with Swerling 1 fluctuation. Fixed threshold shows the worst performance for all three waveforms because it cannot adapt to the amplitude variations introduced by the Swerling model. PhaseCode shows slightly elevated false alarm rates due to its sidelobe structure which penalises its overall score despite having the best PSL of all three waveforms."

#### Plot 6 — Computation Time Comparison

Shows mean processing time per trial for each configuration. Fixed threshold should be fastest. OS-CFAR slowest due to the sort operation.

### What the Results Tell You

#### What Always Holds True Regardless of Trial-to-Trial Variation

These results should be consistent across every run of the script:

- Fixed threshold always underperforms all three CFAR methods in Pd

- PSL is always −13 dB for LFM, −22.3 dB for PhaseCode, ~−18 dB for Hybrid

- SNR gain is always approximately 10 dB for all configurations

- OS-CFAR is always the slowest detector

#### What Varies Between Runs

Because Swerling 1 uses random amplitude fluctuation and AWGN uses fresh random noise each trial, the exact Pd and Pfa values will be slightly different each time you run the script. With 30 Monte Carlo trials the standard deviation of the Pd estimate is approximately sqrt(Pd×(1-Pd)/30) ≈ 0.09 for Pd = 0.75. So expect values within ±0.09 of the true mean. The relative ranking between configurations should remain stable — if CA-CFAR beats GO-CFAR in one run it should beat it in all runs.

### How to Explain This File in the Viva

Q: What is the purpose of generate_comparison_table.m?

It evaluates all 12 combinations of 3 waveforms and 4 detectors under identical controlled conditions across 30 Monte Carlo trials each. It measures 6 performance metrics for every combination and produces a heatmap that visually ranks all configurations simultaneously. The result is the final evidence-based recommendation for which waveform-detector combination is best for our radar system parameters.

Q: Why 30 Monte Carlo trials? Why not just one?

A single trial with Swerling 1 fluctuation gives one random amplitude realisation. One trial might give a particularly bright or dim target. 30 trials average out the random fluctuation and give a statistically representative Pd estimate. With 30 trials the standard error is approximately 0.09 — adequate for distinguishing configurations that differ by 0.1 or more in Pd. More trials would give tighter estimates but the script already takes 4–6 minutes. This is the standard engineering trade-off between accuracy and runtime.

Q: Why is the overall score formula Pd − 10×Pfa and not just Pd?

Because optimising Pd alone is trivial — set alpha = 0 and the threshold is zero, everything is detected, Pd = 1.00. But Pfa would be 1.0 also — every noise cell triggers an alarm. A useful detector must balance both. The factor of 10 on Pfa reflects the operational reality that false alarms are expensive — they trigger unnecessary responses, waste resources, and reduce operator trust in the system. A configuration with slightly lower Pd but much lower Pfa is often preferable in practice. The factor of 10 is one reasonable choice — different applications might use different weights.

Q: What does the heatmap tell you that the bar charts do not?

The bar charts show each metric separately — you see Pd in one chart, Pfa in another, and must mentally combine them. The heatmap shows the combined score in one view. You can instantly see which configurations are consistently good (bright across the heatmap row or column) and which are consistently poor (dark). The heatmap also makes it easy to spot interactions — perhaps GO-CFAR is generally mediocre but specifically good with PhaseCode. The bar charts would require you to look at two separate charts to notice this. The heatmap shows it in one cell.

Q: Why does PSL appear the same for all four detectors within each waveform group?

Because PSL is a property of the waveform autocorrelation — it is computed from xcorr(transmitted, transmitted) before any detection runs. The detector choice does not affect the waveform structure or its autocorrelation sidelobe levels. PhaseCode will always have PSL = −22.3 dB regardless of whether you use CA-CFAR or fixed threshold to detect targets. The four identical PSL bars within each waveform group confirm this correctly — if they differed, it would indicate a measurement error.