Got it! Here's a thorough breakdown of everything in this CEP.

---

## What is this CEP?

**CEP** stands for **Complex Engineering Problem** — it's a major project assignment (worth **15% of your grade**, marked out of **50**) for your **CE363 - Digital Signal Processing** course at **GIK (Ghulam Ishaq Khan Institute)**, Spring 2026, under **Dr. Muhammad Hanif**.

It is mapped to:

- **CLO-2 / PLO-3** → Bloom's Level **C5 (Evaluation/Synthesis)** — meaning you need to critically analyze and compare approaches, not just apply them.

- **CLO-3 / PLO-5** → Bloom's Level **P4 (Articulation)** — meaning you need to practically implement and demonstrate the system.

---

## The Core Problem

You are building a **MATLAB-based radar signal processing system** that can **detect targets** (objects like aircraft, vehicles, etc.) embedded in **noise and clutter**. The challenge is that real radar signals are messy — noise degrades them, multiple targets appear close together, and the environment constantly changes.

The key point the CEP emphasizes: **there is no single correct solution**. You are expected to explore multiple approaches, compare them, justify your choices, and show you understand the trade-offs.

---

## What You Need to Build — The Processing Chain

Think of it as a pipeline with several stages, each of which has multiple design options you must explore:

**1. Waveform Design**

You choose what kind of radar pulse to transmit. Options include:

- **LFM Chirp** (Linear Frequency Modulated) — frequency sweeps across a range

- **Phase-coded waveforms** — signal phase changes in a pattern

- **Hybrid waveforms** — a mix of both

You must evaluate their **ambiguity functions** (which tell you range resolution and sidelobe behavior).

**2. Matched Filtering / Pulse Compression**

The received echo is correlated with the transmitted signal to maximize the SNR. You must implement this and potentially explore alternatives to standard matched filtering.

**3. Windowing**

Before doing frequency analysis, you apply a window function to control **spectral leakage vs. resolution trade-offs** (e.g., Hamming, Hanning, Blackman windows). You need to compare different windowing strategies.

**4. Clutter Mitigation**

"Clutter" is unwanted reflections (from ground, rain, buildings). You must explore:

- **MTI (Moving Target Indicator)** filtering

- **Adaptive filtering**

- **Doppler-based discrimination**

**5. Target Detection with Thresholding**

This is where the system decides "is there a target here or not?". You must compare:

- **Fixed thresholding** (simple, but fragile)

- **CA-CFAR** (Cell Averaging Constant False Alarm Rate)

- **OS-CFAR** (Ordered Statistics CFAR)

- **GO-CFAR** (Greatest Of CFAR)

- Potentially hybrid or data-driven schemes

You need to analyze **Pd (Probability of Detection)** and **Pfa (Probability of False Alarm)** trade-offs.

---

## Scenarios Your System Must Handle

Your simulation environment must be flexible enough to model:

- Multiple **closely spaced targets** (near the resolution limit)

- **Fluctuating target amplitudes** (Swerling models — a statistical model for how target radar cross-section varies)

- **Range-Doppler coupling** (motion affects range measurement)

- **Time-varying clutter** (non-stationary noise)

- **Low SNR stress tests**

- **Mismatched filter conditions**

---

## What You Must Submit (5 Deliverables)

| # | Deliverable | Details |

|---|-------------|---------|

| a | **MATLAB Code** | Modular, configurable, well-commented. Must support multiple design approaches, not just one fixed solution. |

| b | **Simulation Outputs** | Plots and results showing system behavior under different conditions and design choices. |

| c | **Technical Report (IEEE Format)** | Full report with introduction, literature review, block diagrams, math (Z-transform, FFT, detection theory), design justifications, trade-off analysis, results, sensitivity studies, limitations, and IEEE citations. |

| d | **Presentation + Live Demo** | Slides explaining your system design and results, plus a **live MATLAB demonstration** showing real-time effects of changing parameters. |

| e | **Supporting Files** | All figures, parameter tables, comparison charts, datasets, and sensitivity analysis scripts. |

---

## How It's Marked (50 Marks Total)

| Component | Marks |

|-----------|-------|

| Understanding of DSP Concepts | /10 |

| MATLAB Implementation | /10 |

| Desired Outputs & Results | /10 |

| Documentation / Report | /10 |

| Presentation | /10 |

---

## Key Prerequisite Knowledge You Need

You're expected to already know: Sampling Theory, LTI Systems, Convolution & Correlation, Fourier & Z-Transforms, FIR & IIR Filters, FFT, Matched Filtering, and the MATLAB DSP Toolbox.

---

## The Bottom Line

This CEP is essentially asking you to **design, build, test, and justify a complete radar signal processing chain in MATLAB**. The hardest part isn't just coding — it's demonstrating **critical engineering reasoning**: showing that you understand *why* one approach is better than another in given conditions, and quantifying those differences with metrics like SNR gain, Pd/Pfa, resolution limits, and computational complexity.

Would you like help getting started on any specific part — the MATLAB code structure, the report, or understanding any of the DSP concepts?

what are the 3 waveform types explain and tell which is better and how all 3 work

% =========================================================

%  main_radar.m

%  CE363 - Digital Signal Processing | Complex Engineering Problem

%  Radar DSP Processing Framework

% =========================================================

clc; clear; close all;

% ---------------------------------------------------------

%  SECTION 1: SYSTEM PARAMETERS

%  Change these values to explore different radar scenarios

% ---------------------------------------------------------

fs   = 10e6;        % sampling frequency: 10 MHz

T    = 10e-6;       % pulse duration: 10 microseconds

B    = 1e6;         % bandwidth: 1 MHz

c    = 3e8;         % speed of light (m/s)

SNR_dB = 10;        % signal to noise ratio in dB (lower = more noise)

% ---------------------------------------------------------

%  SECTION 2: WAVEFORM SELECTION

%  Change waveform_type to: 'LFM', 'PhaseCode', or 'Hybrid'

% ---------------------------------------------------------

waveform_type = 'LFM';   % <-- CHANGE THIS LINE to switch waveform

if strcmp(waveform_type, 'LFM')

transmitted = generate_LFM(T, B, fs);

elseif strcmp(waveform_type, 'PhaseCode')

transmitted = generate_PhaseCode(fs, T);

elseif strcmp(waveform_type, 'Hybrid')

transmitted = generate_Hybrid(T, B, fs);

else

error('Unknown waveform type. Use LFM, PhaseCode, or Hybrid.');

end

fprintf('Waveform selected: %s\n', waveform_type);

fprintf('Pulse duration: %.1f us | Bandwidth: %.1f MHz | fs: %.1f MHz\n', ...

T*1e6, B/1e6, fs/1e6);

% ---------------------------------------------------------

%  SECTION 3: TARGET SETUP

%  User enters how many targets and their ranges

% ---------------------------------------------------------

num_targets = input('\nEnter number of targets: ');

target_ranges = zeros(1, num_targets);  % range of each target in metres

target_amplitudes = zeros(1, num_targets); % how strong each echo is

for i = 1 : num_targets

target_ranges(i) = input(sprintf('  Enter range of target %d (metres): ', i));

target_amplitudes(i) = input(sprintf('  Enter amplitude of target %d (0.1 to 1.0): ', i));

end

fprintf('\nTargets defined:\n');

for i = 1 : num_targets

fprintf('  Target %d: Range = %.0f m | Amplitude = %.2f\n', ...

i, target_ranges(i), target_amplitudes(i));

end

% ---------------------------------------------------------

%  SECTION 4: SIMULATE RECEIVED SIGNAL

%  Build the received signal = echoes + noise + clutter

% ---------------------------------------------------------

pulse_length  = length(transmitted);

% total receive window: long enough to capture echoes from all targets

max_range     = max(target_ranges);

max_delay_samples = round(2 * max_range * fs / c);

receive_length = pulse_length + max_delay_samples + round(fs * 50e-6); % extra buffer

received = zeros(1, receive_length);  % start with empty received signal

% --- Add each target echo ---

for i = 1 : num_targets

delay_samples = round(2 * target_ranges(i) * fs / c); % two-way travel time

echo = target_amplitudes(i) * transmitted;             % attenuated copy

% place echo at the correct delay position in received signal

received(delay_samples+1 : delay_samples+pulse_length) = ...

received(delay_samples+1 : delay_samples+pulse_length) + echo;

end

% --- Add AWGN noise ---

signal_power = mean(abs(received).^2);

SNR_linear   = 10^(SNR_dB/10);

noise_power  = signal_power / SNR_linear;

noise        = sqrt(noise_power/2) * (randn(1, receive_length) + 1j*randn(1, receive_length));

received     = received + noise;

% --- Add simple clutter (low frequency interference) ---

% Clutter simulated as a slow-varying sinusoidal disturbance

t_receive    = (0 : receive_length-1) / fs;

clutter      = 0.3 * exp(1j * 2 * pi * 0.05e6 * t_receive) .* ...

(0.5 + 0.5*randn(1, receive_length)); % amplitude varies randomly

received     = received + clutter;

fprintf('\nReceived signal simulated with:\n');

fprintf('  SNR = %d dB | Clutter added\n', SNR_dB);

% ---------------------------------------------------------

%  SECTION 5: MATCHED FILTERING (CORRELATION)

%  Correlate received signal with transmitted signal

%  to find where the echoes are (the peaks)

% ---------------------------------------------------------

% Matched filter = cross-correlation in frequency domain (faster method)

N = length(received) + length(transmitted) - 1;  % full correlation length

mf_output = ifft( fft(received, N) .* conj(fft(transmitted, N)) );

mf_output = abs(mf_output);  % take magnitude

% Convert sample index to range in metres

sample_indices = (0 : N-1);

range_axis     = (sample_indices * c) / (2 * fs);  % convert samples to metres

fprintf('\nMatched filtering complete.\n');

% ---------------------------------------------------------

%  SECTION 6: RESULTS AND COMPARISON PLOTS

% ---------------------------------------------------------

figure('Name', sprintf('Radar Processing Results - %s Waveform', waveform_type), ...

'NumberTitle', 'off', 'Position', [100 100 1200 800]);

% --- Plot 1: Transmitted waveform (real part) ---

subplot(3, 2, 1);

t_pulse = (0 : pulse_length-1) / fs * 1e6;   % time in microseconds

plot(t_pulse, real(transmitted), 'b', 'LineWidth', 1.2);

title(sprintf('Transmitted Signal — %s', waveform_type));

xlabel('Time (µs)');

ylabel('Amplitude');

grid on;

% --- Plot 2: Received signal (noisy + clutter) ---

subplot(3, 2, 2);

t_receive_plot = (0 : receive_length-1) / fs * 1e6;

plot(t_receive_plot, real(received), 'r', 'LineWidth', 0.8);

title('Received Signal (with Noise + Clutter)');

xlabel('Time (µs)');

ylabel('Amplitude');

grid on;

% --- Plot 3: Matched filter output — full range ---

subplot(3, 2, 3);

plot(range_axis/1000, mf_output, 'k', 'LineWidth', 1.2);

title('Matched Filter Output (Full Range)');

xlabel('Range (km)');

ylabel('Correlation Amplitude');

grid on;

% Mark the expected target positions

hold on;

for i = 1 : num_targets

xline(target_ranges(i)/1000, '--g', sprintf('T%d', i), 'LineWidth', 1.5);

end

% --- Plot 4: Matched filter output — zoomed around targets ---

subplot(3, 2, 4);

zoom_margin = 500;   % show 500m around min and max target range

x_min = max(0, min(target_ranges) - zoom_margin);

x_max = max(target_ranges) + zoom_margin;

plot(range_axis/1000, mf_output, 'k', 'LineWidth', 1.2);

xlim([x_min/1000, x_max/1000]);

title('Matched Filter Output (Zoomed on Targets)');

xlabel('Range (km)');

ylabel('Correlation Amplitude');

grid on;

hold on;

for i = 1 : num_targets

xline(target_ranges(i)/1000, '--g', sprintf('T%d = %.0fm', i, target_ranges(i)), ...

'LineWidth', 1.5);

end

% --- Plot 5: SNR comparison before and after matched filtering ---

subplot(3, 2, 5);

snr_before = SNR_dB;

% estimate SNR after: peak power vs noise floor power

peak_val   = max(mf_output);

% noise floor estimated from first 100 samples (before any echo)

noise_floor = mean(mf_output(1:100));

snr_after  = 20 * log10(peak_val / noise_floor);

bar_labels = categorical({'Before MF (dB)', 'After MF (dB)'});

bar(bar_labels, [snr_before, snr_after], 0.4, 'FaceColor', [0.2 0.6 0.8]);

title('SNR: Before vs After Matched Filtering');

ylabel('SNR (dB)');

grid on;

text(1, snr_before + 1, sprintf('%.1f dB', snr_before), 'HorizontalAlignment', 'center');

text(2, snr_after  + 1, sprintf('%.1f dB', snr_after),  'HorizontalAlignment', 'center');

% --- Plot 6: Normalised MF output in dB (shows sidelobes clearly) ---

subplot(3, 2, 6);

mf_norm_dB = 20 * log10(mf_output / max(mf_output) + eps); % normalised to 0 dB peak

plot(range_axis/1000, mf_norm_dB, 'm', 'LineWidth', 1.2);

ylim([-60 5]);

title('Matched Filter Output — Normalised (dB scale)');

xlabel('Range (km)');

ylabel('Amplitude (dB)');

grid on;

hold on;

yline(-13, '--r', '-13 dB sidelobe ref', 'LineWidth', 1);

for i = 1 : num_targets

xline(target_ranges(i)/1000, '--g', sprintf('T%d', i), 'LineWidth', 1.5);

end

% ---------------------------------------------------------

%  SECTION 7: PRINT SUMMARY TO COMMAND WINDOW

% ---------------------------------------------------------

fprintf('\n========== RESULTS SUMMARY ==========\n');

fprintf('Waveform        : %s\n', waveform_type);

fprintf('No. of Targets  : %d\n', num_targets);

fprintf('SNR (before MF) : %.1f dB\n', snr_before);

fprintf('SNR (after MF)  : %.1f dB\n', snr_after);

fprintf('SNR Gain        : %.1f dB\n', snr_after - snr_before);

fprintf('\nTarget Detection Results:\n');

[peaks, peak_locs] = findpeaks(mf_output, 'MinPeakHeight', max(mf_output)*0.3, ...

'MinPeakDistance', round(50*fs/c));

detected_ranges = range_axis(peak_locs);

for i = 1 : length(detected_ranges)

fprintf('  Detected peak %d at range: %.1f m\n', i, detected_ranges(i));

end

fprintf('======================================\n');